High-Velocity Native Audio Architecture and Gated-Cascade Pitch Detection Engine for React Native Complete Unabridged Engineering Specification: Baseline v1.0, v1.1 Split-Scale, v1.2 Dynamic Production & v2.0 Apex Architecture

1. End-to-End Latency Architecture: Microphone to Detected Note

Achieving ultra-low-latency acoustic note detection within a React Native application requires eliminating the JavaScript runtime from the real-time signal processing path [1]. Standard React Native audio libraries serialize Pulse-Code Modulation (PCM) buffers across the asynchronous bridge or pass typed arrays through the JavaScript Interface (JSI) at frame rate, subjecting time-critical spectral analysis to garbage collection pauses, UI rendering stalls, and event-loop contention [2]. To satisfy the strict requirement of capturing microphone data at the fastest physical rate and resolving pitch events in minimal time, the system must execute the entire pipeline—from hardware analog-to-digital conversion (ADC) to symbolic note onset confirmation—inside a shared, zero-allocation C++17 engine exposed via a React Native New Architecture Pure C++ TurboModule [1].

The optimal architecture partitions the real-time workload into four isolated execution domains: hardware DMA capture, lock-free inter-thread transport, multi-rate gated-cascade signal analysis, and asynchronous JSI event dispatch [1]. Audio samples never cross into the JavaScript thread; instead, the native engine processes continuous 48 kHz audio in 256-sample hops (≈ 5.33 ms or 5333 µs per hop) and wakes the JavaScript thread exclusively upon confirmed state transitions (noteOn and noteOff) [1].

| Data Flow Stage | Processing Component | Execution Domain | Latency / Hop Budget | Primary Mechanism |
| --- | --- | --- | --- | --- |
| Microphone ADC | Hardware Audio Codec | Hardware / Kernel DMA | 1.0 – 2.6 ms | Low-latency audio driver ring [1] |
| Stage 1: Capture | OboeInput / IosAudioInput | OS Real-Time Audio Callback | < 0.1 ms | Exclusive unmixed DMA capture [1] |
| Stage 2: Transport | SampleRing<16384> | Lock-Free Inter-Thread Buffer | 50 – 70 ns | Wait-free SPSC with alignas(64) & zero-drift fast-forward [1, 6, 7] |
| Stage 3: DSP Core | Cascade::processHop | Dedicated Native Worker Thread | 0.4 µs idle / 12–35 µs active | Multi-rate inharmonic 4x-SIMD Goertzel filterbank [1] |
| Stage 4: Bridge | NativePianoEngine | React Native JSI Callback | < 0.2 ms | C++ AsyncCallback to JS runtime [1] |

Stage 1: Hardware-Direct Microphone Acquisition

The primary bottleneck in mobile acoustic capture is the operating system's internal audio mixer and voice preprocessing pipeline [4]. By default, mobile operating systems route microphone input through Automatic Gain Control (AGC), echo cancellation, and voice-optimized band-pass filters, introducing tens of milliseconds of buffering while attenuating fundamental frequencies below 100 Hz [5].

On Android, the fastest physical path from the hardware ADC to application memory is Google's Oboe C++ library (OboeInput.cpp), which wraps AAudio on modern devices and falls back to OpenSL ES on legacy systems [1]. Configuring oboe::AudioStreamBuilder with oboe::PerformanceMode::LowLatency, oboe::SharingMode::Exclusive, and oboe::InputPreset::Unprocessed instructs the Android audio HAL to bypass the system mixer entirely, mapping a shared memory DMA ring buffer directly between the audio driver and the application process [4].

On iOS, the equivalent bare-metal path utilizes the RemoteIO AudioUnit (IosAudioInput.mm) coupled with AVAudioSession configured under AVAudioSessionModeMeasurement [1]. Activating AVAudioSessionModeMeasurement is essential for musical instrument analysis because it disables Apple's built-in dynamic range compression and high-pass filtering, preserving the phase and amplitude integrity of low piano fundamentals down to A0 (27.5 Hz) while permitting hardware I/O buffer durations as low as 128 or 256 samples [1].

Stage 2: Lock-Free Cache-Aligned SPSC Transport

Because the OS audio callback executes on a high-priority real-time kernel thread, executing Fast Fourier Transforms (FFTs), autocorrelation, or heap allocations inside the callback risks buffer underruns (lateHops) and operating system watchdog termination [1]. Protecting a shared sample buffer with a standard std::mutex is equally viability-breaking, as mutex contention introduces priority inversion and kernel context-switch overhead [11].

The optimal transport mechanism between the OS audio callback (producer) and the C++ DSP analysis thread (consumer) is a wait-free Single-Producer Single-Consumer (SPSC) Ring Buffer (SampleRing) [1]. In an SPSC topology, only the producer modifies the write index (m_tail / tail_) and only the consumer modifies the read index (m_head / head_), eliminating the need for read-modify-write atomic instructions like compare_exchange [7]. Concurrency correctness without locks is achieved through strict C++11/C++17 acquire-release atomic memory ordering [1]. Specifically, each thread loads its own index using std::memory_order_relaxed since no other thread mutates that variable, while reading the opposite thread's index via std::memory_order_acquire to determine available buffer capacity [7]. After writing or reading the PCM samples, the thread publishes its updated index using std::memory_order_release, which guarantees that all prior non-atomic writes to the sample array become visible to the consumer before the index update is observed, avoiding the full pipeline flush of std::memory_order_seq_cst [7].

Hardware-level mechanical sympathy further dictates that the atomic indices be isolated across CPU cache lines [6]. Modern ARM64 and x86-64 processors manage L1/L2 cache coherency in 64-byte cache lines via the MESI protocol. If head_ and tail_ reside adjacent to one another in memory, they occupy the same 64-byte cache line, causing every sample push from the microphone core to invalidate the cache line on the DSP analysis core and forcing a cross-core bus synchronization stall known as cache false sharing [7]. Padding both atomic indices with alignas(64) (or std::hardware_destructive_interference_size) isolates each variable onto a dedicated cache line, empirically increasing SPSC queue throughput by 33.7% (from 29.16 million to 38.99 million elements per second) and reducing inter-core one-way transfer latency to 50–70 ns [6]. Furthermore, constraining the ring buffer capacity N to a power of two (16384) replaces costly integer modulo division ((tail + 1) % N) and conditional wrap-around branches with a single-cycle bitwise AND mask ((tail + 1) & (N - 1)).

2. Algorithmic Evaluation: Why Gated-Cascade DSP Outperforms Neural AMT and Fixed-Window Pitch Trackers

Selecting the core detection engine requires navigating a fundamental physical constraint: the Heisenberg-Gabor Uncertainty Principle, which dictates an inverse relationship between frequency resolution and temporal responsiveness [16]. In musical acoustics, pitch spacing is logarithmic, meaning the frequency separation between adjacent semitones shrinks drastically in the bass register [10]. On an 88-key piano (A0 at MIDI 21, 27.5 Hz to C8 at MIDI 108, 4186 Hz), the semitone gap between A0 (27.50 Hz) and A#0 (29.14 Hz) is only 1.64 Hz, whereas a single cycle of A0 takes 36.36 ms (1745 samples at 48 kHz) [10].

Heisenberg-Gabor Time-Frequency Uncertainty: Δf · Δt ≥ 1 / (4π) Equal-Temperament Pitch Spacing: f₀(p) = 440 × 2^((p − 69) / 12) Hz

The Failure of Fixed-Window Autocorrelation and Neural AMT in Real-Time Streaming

Time-domain autocorrelation methods—such as YIN, pYIN, and the McLeod Pitch Method (MPM)—estimate fundamental frequency by finding the lag τ that maximizes the normalized square difference function across an analysis window W, requiring at least two full waveform periods (W ≥ 2τ) for reliable periodicity detection [1]. Evaluating fixed-window MPM across all 88 keys at 48 kHz reveals an irreconcilable trade-off between bass register accuracy and CPU efficiency [1]. When configured with a fixed 1024-sample window (21.3 ms), MPM executes rapidly at 37 µs per hop and achieves 100% accuracy (61/61) across the mid and treble registers (C3–C8), but fails catastrophically in the bass register (A0–B2), detecting only 6 of 27 keys (22.2%) because a 21.3 ms window cannot contain two full periods of any frequency below 93.75 Hz (F#2) [1]. Expanding the tracker to a fixed 4096-sample window (85.3 ms) successfully resolves all 27/27 bass keys alongside all 61/61 upper keys, but increases mean active CPU consumption more than fivefold to 186 µs per hop while smearing rapid treble note repetitions across an 85.3 ms temporal lookback [1].

Concurrently, deep learning Automatic Music Transcription (AMT) models are structurally unsuited for sub-20 ms interactive mobile feedback [19]. Spotify's widely used Basic Pitch model relies on a lightweight Convolutional Neural Network (CNN) operating over a harmonic Constant-Q Transform (CQT) followed by non-causal post-processing [20]. Empirical evaluations confirm that Basic Pitch is architecturally incompatible with real-time streaming inference [19]; even when exported to ONNX Runtime on CPU, it consumes 951 MB of RAM, runs at only 19.4× real-time in batch mode, and achieves a modest 0.578 Frame F1 score [20]. While recent architectures like Harmonica-nano (26.3K parameters, combining ResNet blocks, multi-depth harmonic convolutions, and frequency-grouped LSTMs) improve accuracy to 0.796 Frame F1 and accelerate batch speed to 1622.5× real-time, they still depend on a fixed 32 ms CQT frame interval with three bins per semitone on 16 kHz audio [20]. That 32 ms frontend buffering interval alone is nearly five times slower than the 6.7 ms total strike-to-NoteOn treble latency achieved by a native multi-rate DSP cascade [1].

The Gated-Cascade Architecture (piano-engine)

To achieve optimal speed and accuracy simultaneously across all 88 keys, the piano-engine architecture implements a Four-Stage Gated Cascade (Cascade::processHop) that dynamically adapts its time-frequency resolution to the incoming acoustic signal [1].

In the first stage, a time-domain energy gate and spectral onset detector evaluate root-mean-square (RMS) energy and spectral flux every 256-sample hop (5.33 ms) against background noise thresholds [1]. During silence or ambient room noise—verified empirically at -60, -40, and -30 dBFS—the gate short-circuits immediately in 0.4 µs, consuming less than 0.008% of the 5333 µs hop budget while generating zero false triggers [1]. Incorporating Per-Channel Energy Normalization (PCEN)—which applies an adaptive exponential moving average automatic gain control across frequency subbands prior to dynamic range compression—further stabilizes onset detection against room reverberation and microphone distance variations [19].

When an onset or active sustain passes the gate, the second stage executes multi-window escalating pitch estimation and targeted Goertzel evaluation rather than defaulting to a costly 4096-sample window [1]. Using warm tracking and progressive window escalation, the detector first evaluates periodicity across a compact window (512–1024 samples) to resolve treble and mid-register notes (C4–C8) within 4–18 ms in real-time threaded operation (6.7 ms p50 treble and 12 ms mid-register in synthetic benchmarks) [1]. Only when low-frequency energy is present does the cascade use 2048- and 4096-sample ring-buffer lookbacks to resolve lower-mid (C3–B3 in 23 ms) and deep bass notes (A0–B2 in 44 ms p50, 92 ms p95) [1]. Because most hops either terminate at the idle gate or lock onto a warm-tracked pitch, the median (p50) active compute cost is just 4 µs (mean 30 µs, p99 327 µs) [1].

The third stage performs harmonic partial verification and accounts for piano string inharmonicity [1]. Real piano strings behave as stiff mechanical resonators whose overtone partials deviate progressively sharper than integer multiples of the fundamental frequency f₀ according to Fletcher's inharmonicity law [1, 9]:

Fletcher's Piano String Inharmonicity Law: f_h = h · f₀ · √(1 + B · h²), where B = (π³ · E · d⁴) / (64 · l² · T) Iterative Phasor / Goertzel Resonator: R(ω_k, t) = (1 − α) · R(ω_k, t − Δt) + α · x(t) · e^(−i·ω_k·t)

Here, h represents the partial index, B is the inharmonicity factor, E is Young's modulus, d is string diameter, l is speaking length, and T is string tension [9]. Because time-domain autocorrelation (MPM) locks onto the composite period of all active harmonics, this upward stretching of upper partials pulls MPM's raw pitch estimate sharp of the true fundamental by a median of +12 to +19 cents on grand pianos and +27 to +43 cents on shorter, stiffer upright strings [1]. Stage 3 evaluates targeted Goertzel single-bin discrete Fourier resonators at the candidate fundamental and its expected stretched partials to verify genuine string excitation, suppress subharmonic octave errors, and enforce a 2-hop confirmation before emitting noteOn [1].

The fourth stage manages polyphonic chord detection through two distinct operational modes tailored to application context [1]. In interactive music education and score-following scenarios where the application knows which note or chord the user should play next, Score-Informed Expected-Notes Mode (setExpectedNotes([midi...])) restricts Stages 2 and 3 to evaluating targeted Goertzel and comb resonators specifically at the expected fundamentals and their partials [1]. When expecting a C major triad {C4, E4, G4} ([60, 64, 67]), striking E4 alone triggers only E4, playing adjacent semitones (D#4, F4) or unrelated pitches (A3, C5) triggers nothing, and striking the full triad resolves all three notes in 10.6–33 ms [1]. Conversely, when unguided free-form chord detection is required, Blind Polyphonic Mode activates an iterative harmonic summation and cancellation scheme derived from Klapuri's multiple-F0 estimator [1, 29]. The input spectrum X(k) is whitened across subbands using a compression exponent ν = 0.33 to obtain Y(k) = γ(k)·X(k), and the salience s(τ) of each candidate period τ is computed by summing weighted harmonic partials [29]:

Klapuri Multi-F0 Harmonic Salience & Iterative Spectral Cancellation: s(τ) = ∑_{m=1..M} g(τ, m) · max_{k ∈ κ(τ,m)} ¦Y_R(k)¦, with g(τ, m) = (f_s/τ + α) / (m · f_s/τ + β) Residual Spectrum Cancellation: Y_R(k) ← max(0, Y_R(k) − d · Y_D(k))

3. Production Engine v1.0, v1.1, and v1.2 Specifications (Historical Baseline & Evolution)

Production Engine v1.0 Baseline Architecture

The v1.0 piano-engine baseline provides the foundational low-latency plumbing for React Native New Architecture (React Native 0.82+ through 0.87) using Pure C++ TurboModules [1].

| Source Path | Target Application Path | Architectural Responsibility (v1.0 – v2.0) |
| --- | --- | --- |
| shared/SampleRing.h | <app>/shared/SampleRing.h | Cache-aligned (alignas(64)) lock-free SPSC ring buffer for real-time PCM transport [1] |
| shared/Cascade.h, Calibration.h | <app>/shared/ | Portable C++17 DSP core implementing the gated cascade, multi-rate Goertzel resonators, Klapuri polyphony, and two-strike calibration [1] |
| shared/Engine.h, AudioInput.h, NativePianoEngine.{h,cpp} | <app>/shared/ | Analysis worker thread, hardware abstraction interface, and JSI C++ TurboModule binding [1] |
| specs/NativePianoEngine.ts | <app>/specs/NativePianoEngine.ts | React Native Codegen TypeScript specification (filename must begin with Native) [1] |
| src/CalibrationScreen.tsx, src/pianoEngine.ts | <app>/src/ | Guided Two-Strike React Native calibration UI, usePianoDetector() hook, and telemetry display [1] |
| android/app/src/main/jni/OboeInput.cpp, CMakeLists.txt | <app>/android/app/src/main/jni/ | Android low-latency Oboe microphone capture and NDK CMake build script [1] |
| ios/IosAudioInput.mm, NativePianoEngineProvider.{h,mm} | <app>/ios/<AppName>/ | iOS RemoteIO AudioUnit microphone capture and Objective-C++ TurboModule provider [1] |
| test/run.sh | <app>/test/run.sh | Standalone desktop C++17 benchmark (./run.sh quick¦full¦upright) for accuracy, CPU, and threaded pacing [1] |

v1.0 Baseline Implementation Reference (shared/SampleRing.h & shared/Cascade.h)

shared/SampleRing.h (v1.0 Baseline)

// shared/SampleRing.h (v1.0 Baseline) #pragma once #include <atomic> #include <cstddef> #include <cstdint> #include <cstring> #include <algorithm> namespace piano { template <std::size_t CapacityPow2 = 16384> class SampleRing { static_assert((CapacityPow2 & (CapacityPow2 - 1)) == 0, "CapacityPow2 must be a power of two"); static constexpr std::size_t kMask = CapacityPow2 - 1; public: SampleRing() = default; std::size_t push(const float* data, std::size_t count) noexcept { const std::size_t tail = tail_.load(std::memory_order_relaxed); const std::size_t head = head_.load(std::memory_order_acquire); const std::size_t avail = CapacityPow2 - (tail - head); const std::size_t toWrite = std::min(count, avail); for (std::size_t i = 0; i < toWrite; ++i) { buffer_[(tail + i) & kMask] = data[i]; } tail_.store(tail + toWrite, std::memory_order_release); return toWrite; } std::size_t available() const noexcept { const std::size_t tail = tail_.load(std::memory_order_acquire); const std::size_t head = head_.load(std::memory_order_relaxed); return tail - head; } void peekWindow(float* dst, std::size_t windowSize) const noexcept { const std::size_t head = head_.load(std::memory_order_relaxed); const std::size_t start = head - windowSize; for (std::size_t i = 0; i < windowSize; ++i) { dst[i] = buffer_[(start + i) & kMask]; } } void consume(std::size_t hopSize) noexcept { const std::size_t head = head_.load(std::memory_order_relaxed); head_.store(head + hopSize, std::memory_order_release); } private: alignas(64) std::atomic<std::size_t> head_{4096}; alignas(64) std::atomic<std::size_t> tail_{4096}; alignas(64) float buffer_[CapacityPow2]{}; }; } // namespace piano

shared/Cascade.h (v1.0 Baseline)

// shared/Cascade.h (v1.0 Baseline) #pragma once #include <cmath> #include <cstdint> #include <vector> namespace piano { struct NoteEvent { enum Type : uint8_t { NoteOn = 1, NoteOff = 0 } type; int midi; float cents; float velocity; double tMs; }; struct Config { int sampleRate = 48000; int hopSize = 256; // 5.333 ms per hop @ 48 kHz float gateRmsDb = -48.0f; // Rejects ambient noise (-60/-40/-30 dBFS) float onsetFluxRatio = 1.85f; // Spectral/energy rise ratio for attack float mpmClarityShort = 0.88f; // Threshold to accept 1024-sample MPM float mpmClarityLong = 0.80f; // Threshold for escalated 4096-sample MPM float goertzelVerifyRatio = 0.12f; // Min partial energy relative to total frame int confirmHops = 2; // 2-hop debounce for zero false positives bool polyphonyEnabled = false; // Stage 4 experimental chord estimator }; class Goertzel { public: static float magnitudeSquared(const float* x, int N, float fHz, int sr) noexcept { const float omega = 2.0f * static_cast<float>(M_PI) * fHz / static_cast<float>(sr); const float coeff = 2.0f * std::cos(omega); float s0 = 0.0f, s1 = 0.0f, s2 = 0.0f; for (int n = 0; n < N; ++n) { s0 = x[n] + coeff * s1 - s2; s2 = s1; s1 = s0; } return s1 * s1 + s2 * s2 - coeff * s1 * s2; } }; } // namespace piano

Engine v1.1 Specification (Split-Scale Calibration & Whistle Rejection Intermediate Upgrade)

Engine v1.1 introduced the Two-Bridge Split-Scale Inharmonicity Curve across the bridge transition at C3 (MIDI 48), replacing fixed static constants with empirical calibration [1]:

Two-Bridge Split-Scale Inharmonicity Model: ln B_bass(m) = a_B · m + b_B (for m < 48), ln B_treble(m) = a_T · m + b_T (for m ≥ 48) Whistle Rejection Overtone Gate: (E₂ + E₃ + E₄) / E₁ ≥ η_min

Additionally, v1.1 resolved whistle false-positives by enforcing a minimum overtone energy ratio (E₂ + E₃ + E₄)/E₁ ≥ η_min and addressed C-D-C-D trills using a single 1-tap comb filter [1]. However, v1.1 remained limited to monophonic voice tracking (activeMidi_), evaluated dynamic envelope velocity using a single global range (minVelocityDb to maxVelocityDb), and utilized a static timbre template learned exclusively from Strike 2 (firm), causing gentle 5%–25% key presses to be rejected as sine-wave whistles [1].

Engine v1.2 Specification: 5%–100% Dynamic Intensity, Auto-Gain Staging, 6-Voice Polyphony & Damper Articulation

Engine v1.2 introduced auto-calibrated microphone gain staging, dual-profile timbre interpolation for 5% to 100% playing intensity, 6-voice polyphonic tracking with harmonic subtraction, and damper-drop release detection [28, 30, 32]. However, as detailed in Section 4 below, a deep mathematical, physical, and concurrency audit of v1.2 exposed 11 critical real-world failure modes—including a +27.1 dB / +33.1 dB Parseval Goertzel normalization mismatch, pre-gain vs. post-gain calibration threshold shift omission, scalar chord serialization deadlock, rectangular window sidelobe leakage across the C3 bridge break, missing-fundamental bass rejection, premature high-treble damper cutoffs, and non-atomic cross-thread vector mutation—which are completely resolved in the v2.0 Apex Production Architecture.

4. Deep Permutation Audit of v1.2 & Architectural Evolution to v2.0 Apex

To make the piano detector an undisputed industry leader across all acoustic grands, small upright pianos, digital keyboards, and mobile microphone hardware, every playing permutation and edge case was audited against v1.2. The table below documents all 11 failure modes discovered in v1.2 and their exact mathematical and architectural resolutions in v2.0 Apex.

| # | Real-World Playing Permutation | Root Cause in v1.2 (Cascade.h & Calibration.h) | Observed Failure Mode in v1.2 | v2.0 Apex Solution |
| --- | --- | --- | --- | --- |
| 1 | Soft-to-medium notes (5%–70% intensity) | Goertzel e1 divided by (0.5 * N) instead of Hann Parseval factor (0.125 * N²), inflating noteDb by +27.1 dB (N=1024) to +33.1 dB (N=4096) over rmsDb. | normI clamps to 1.0f on every note; velocity is stuck near 100% and soft timbre weights are bypassed. | Exact Parseval Hann power normalization: P = magSq / (0.125 * N²), matching time-domain RMS dBFS. |
| 2 | Device mic requiring digital pre-gain (G_dB ≠ 0 dB) | Calibration.h records keySoftDb, keyLoudDb, and calibratedGateDb on raw pre-gain audio, then computes inputGainDb at the end without shifting thresholds. | Cascade.h amplifies live audio by +inputGainDb while comparing against un-shifted thresholds (up to 24 dB mismatch). | Post-Gain Threshold Alignment in finalizeProfile(): shifts ambientNoiseDb, gate, keySoftDb, and keyLoudDb by +inputGainDb. |
| 3 | Simultaneous block chords (2 to 4 notes struck at t=0) | Stage 3 tracks only a single scalar bestMidi, candidateMidi_, and confirmCount_ per hop. | Chords serialize into arpeggios; two equal-energy chord notes ping-pong candidateMidi_ and deadlock. | Intra-hop iterative Klapuri peeling (up to 4 notes/hop) + per-key confirmation array keyConfirmCount_[128]. |
| 4 | Holding loud bass note (C3) vs. playing C3 + C4 octave | Harmonic subtraction uses a fixed eh - 0.75f * eh (-6 dB) multiplier regardless of active voice partial power. | Loud C3 still has enough 2f₀ energy after -6 dB cut to false-trigger C4; decaying C3 suppresses soft C4. | Physical spectral power subtraction using tracked voice.partialPower[vh] weighted by bin overlap. |
| 5 | Tenor / Bridge-Break octave (C3–B3, MIDI 48–59) | Switches from N=4096 to N=1024 at m=48 (130.8 Hz). At 48 kHz, N=1024 has a 46.88 Hz bin width while C3 to C#3 is 7.78 Hz. | Adjacent semitones bleed into the same main lobe; chromatic passages and minor 2nds blur. | 3-Tier Multi-Rate Windowing: N=4096 (m<42), N=2048 (42≤m<66), and N=1024 (m≥66). |
| 6 | Fast scales, trills, or dissonant intervals (2nds/7ths) | Goertzel runs on unwindowed rectangular buffers (gainWindow4096_), producing -13 dB spectral sidelobes. | Spectral leakage across adjacent keys corrupts estimateFineCents and triggers neighbor keys. | Pre-computed Hann windows (hann4096_, hann2048_, hann1024_) with -31.5 dB sidelobe suppression. |
| 7 | Deep bass notes (A0–G1, MIDI 21–31) on uprights/phones | Candidate gate checks only fundamental e1, which is 15–25 dB weaker than 2f₀, 3f₀ on small soundboards and MEMS mics. | Deep bass notes are ignored or misdetected as +1 octave (A1 instead of A0); C1 calibration fails. | Missing-Fundamental Virtual Pitch gate max(P₁, 0.7(P₂+P₃)) + Odd-Harmonic (h=3,5) Disambiguation bonus. |
| 8 | High treble sustain (E6–C8, MIDI 88–108) | Acoustic pianos have no dampers above ~E6 and strings decay naturally at >2.0 dB/hop; v1.2 fires NoteOff at decaySlope > 1.6f. | High treble notes emit premature NoteOff 21 ms after strike even while key is held. | Register-aware damper slope (3.2 dB/hop for m≥88 vs 1.55 dB/hop for m<88) + 2-hop damperDropHops counter. |
| 9 | Sustain pedal passages exceeding 6 notes | findFreeVoiceSlot() overwrites the oldest voice slot without emitting NoteOff for the evicted note. | Evicted notes remain permanently stuck in NoteOn state in the React Native UI / MIDI stream. | Emit explicit NoteOff prior to voice eviction + expand polyphony capacity to 10 voices. |
| 10 | Fortissimo (ff) percussive hammer strike ('key thump') | Hammer impact creates 1–2 hops of broadband knock and transient pitch sharpness (+20 cents) before standing wave locks. | Pitch window and cent jitter checks reject Hop 1, delaying fortissimo onset by 10–15 ms. | Transient-Aware Pitch Tolerance (transientHopsRemaining_) widens cent tolerance during >6.5 dB/hop attacks. |
| 11 | Calling setExpectedNotes or setProfile during live play | expectedNotes_ = expected mutates a heap std::vector<int> on the JS thread while the DSP worker thread iterates it. | Undefined Behavior / Use-After-Free crash and real-time thread heap lock. | Wait-free 128-bit Atomic Bitmask (expectedMaskLow_, expectedMaskHigh_) + double-buffered PianoProfile swap. |

Cross-Generation Capability Matrix: v1.0 vs. v1.1 vs. v1.2 vs. v2.0 Apex

| Subsystem / Feature | v1.0 Baseline | v1.1 Intermediate | v1.2 Production | v2.0 Apex Architecture |
| --- | --- | --- | --- | --- |
| Microphone Gain & Threshold Alignment | None (Raw hardware float input) [1] | None (Assumed standard iPhone level) | Auto-Gain Normalizer (G_dB ∈ [-12, +24] dB), but thresholds un-shifted [31] | Post-Gain Aligned Auto-Staging: Normalizes firm strikes to -10 dBFS and shifts gate, keySoftDb, and keyLoudDb by +G_dB. |
| Spectral Power Normalization | Unnormalized Goertzel ratio [1] | Unnormalized overtone ratio | Divided by 0.5*N (+27.1 dB to +33.1 dB offset vs RMS dBFS) | Exact Parseval Hann Normalization: P = magSq / (0.125 * N²), matching time-domain RMS dBFS across all windows. |
| Multi-Rate Windowing & Leakage | Rectangular 1024 / 4096 escalation [1] | Rectangular 1024 / 4096 split at C3 | Rectangular N=4096 (m<48) and N=1024 (m≥48); -13 dB sidelobes | 3-Tier Hann Filterbank: N=4096 (m<42), N=2048 (42≤m<66), N=1024 (m≥66) with -31.5 dB sidelobes. |
| Polyphonic Voice & Chord Tracking | Monophonic scalar activeMidi_ [1] | Monophonic scalar with 1-tap comb notch | 6-voice table, but scalar candidateMidi_ serialized chords [28] | 10-Voice Table + Intra-Hop Klapuri Peeling (4 simultaneous onsets/hop) + per-key keyConfirmCount_[128]. |
| Harmonic Spectral Subtraction | None in monophonic path [1] | 1-tap time-domain comb filter | Fixed eh - 0.75*eh (-6 dB) cut regardless of voice level [28] | Physical Partial Power Subtraction: Subtracts tracked voice.partialPower[vh] weighted by bin overlap. |
| Deep Bass (A0–B1) Recovery | Escalated 4096 MPM [1] | Split-scale B curve below C3 | Gated solely on e1; failed on missing-fundamental bass notes | Virtual Pitch Composite Gate max(P₁, 0.7(P₂+P₃)) + Odd-Harmonic (h=3,5) Disambiguation bonus. |
| Note-Off & Voice Stealing | 3-hop global RMS silence timeout [1] | 3-hop global RMS silence timeout | Fixed 1.6 dB/hop slope cut off high treble; voice steal leaked NoteOff [30] | Register-Aware Damper Slope (1.55 vs 3.2 dB/hop) + 2-hop confirmation + clean NoteOff on voice steal. |
| DSP Vectorization & Concurrency | Scalar trig per hop [1] | Scalar trig per hop | Non-atomic std::vector mutation across threads; scalar Goertzel | 4-Way SIMD Goertzel (magnitudeSquared4x), precomputed coeffTable_, 128-bit atomic mask, double-buffered profile. |

5. The Physics and Mathematics of Engine v2.0 Apex

1. Microphone Auto-Gain Staging and Post-Gain Threshold Alignment

Because Android InputPreset::Unprocessed and iOS AVAudioSessionModeMeasurement bypass OS-level AGC, raw signal levels vary by up to 25 dB depending on device hardware, distance to the soundboard, and digital piano volume sliders [1]. During calibration, the engine measures the raw ambient noise floor L_noise,raw and the peak raw level of Strike 2 L_firm,raw. The digital pre-gain factor G_linear is computed to normalize firm strikes to a -10 dBFS operating ceiling, and all calibrated decibel thresholds are shifted by +G_dB so they align with the post-gain signal in Cascade::processHop:

G_dB = clamp(−10.0 − L_firm,raw, −12.0, +24.0), G_linear = 10^(G_dB / 20) y[n] = G_linear · x[n] L_noise = L_noise,raw + G_dB, L_soft(m) = L_soft,raw(m) + G_dB, L_loud(m) = L_loud,raw(m) + G_dB

2. Exact Parseval Windowed Power Normalization

A sinusoidal partial of amplitude A has time-domain mean-square power P_time = A² / 2. When evaluated by a Goertzel resonator of length N multiplied by a Hann window w[n] = 0.5 · (1 − cos(2πn / (N − 1))) with coherent gain W_coh = 0.5, the raw Goertzel magnitude squared at the bin center is |X_Goertzel(ω)|² = (N · W_coh · A / 2)² = N² · A² / 16. Dividing by 0.125 · N² recovers the exact time-domain mean-square power A² / 2 across all window sizes N ∈ {1024, 2048, 4096}:

P_partial(m, h) = ¦X_Goertzel(ω_{m,h})¦² / (0.125 · N²) = A_{m,h}² / 2 L_partial,dB(m, h) = 10 · log₁₀(P_partial(m, h) + 10⁻¹²)

3. Dual-Profile Timbre Interpolation for 5%–100% Intensity

Piano hammer felt behaves as a hysteretic nonlinear spring [9]. On a 5%–20% soft press, hammer-string contact duration is prolonged, transferring energy almost exclusively into the fundamental (h = 1) and second partial (h = 2), while partials h = 3..6 remain near the noise floor. On a 75%–100% hard strike, contact duration shortens, exciting bright upper partials (h = 2..8). During calibration, Strike 1 (soft) populates w_soft and Strike 2 (firm) populates w_loud. During real-time detection, the target timbre template w_target(m, I_norm) is dynamically interpolated based on the normalized strike intensity I_norm ∈ [0.0, 1.0] [30]:

w_target(m, I_norm) = (1 − I_norm) · w_soft(m) + I_norm · w_loud(m) CosineSimilarity(a, w_target) = (∑_{h=1..6} a_h · w_{target,h}) / (√(∑_{h=1..6} a_h²) · √(∑_{h=1..6} w_{target,h}²) + 10⁻⁹)

4. Hybrid Velocity Calculation (Partial Harmonic Energy + Scale-Invariant Brightness)

True musical velocity is computed from two independent acoustic dimensions: normalized partial harmonic energy (I_norm) and scale-invariant harmonic spectral brightness (B_norm) [32]:

I_norm = clamp( (L_note,dB − L_soft(m)) / max(6.0, L_loud(m) − L_soft(m)), 0.0, 1.0 ) B_norm = clamp( (P₂ + P₃ + P₄) / (1.8 · P_fund), 0.0, 1.0 ) Velocity_final = clamp( 0.05 + 0.95 · (0.65 · I_norm + 0.35 · B_norm), 0.05, 1.00 )

5. 10-Voice Polyphonic Tracking, Physical Spectral Subtraction & Intra-Hop Chord Peeling

Before evaluating candidate keys, v2.0 Apex subtracts the tracked physical partial powers P_v(vh) of all currently active voices weighted by spectral bin proximity overlap γ(Δf). Then, inside Stage 4, up to 4 simultaneous chord notes are peeled iteratively within the same hop:

Inter-Hop Active Voice Subtraction: P_{m,res}(h) = max(0, P_m(h) − 1.15 · (1 − 0.5 · ¦f_{m,h} − f_{v,vh}¦ / Δf_bin) · P_v(vh)) Intra-Hop Klapuri Chord Peeling: P_{m,res}(h) ← max(0, P_{m,res}(h) − 1.25 · P_{m*}(wh)) Odd-Harmonic Bass Disambiguation Bonus (m < 48): ρ_odd = (P₃ + P₅) / (P₂ + P₄ + 10⁻¹²) > 0.12 ⇒ Salience × 1.35

6. Register-Aware Damper-Drop Articulation Tracking

When a damped piano key (m < 88) is released, the felt damper drops onto the vibrating string, causing a rapid energy decay (> 29 dB/s, or > 1.55 dB/hop) [30]. Above MIDI 88 (E6–C8), pianos have no dampers and strings decay naturally at 2.0–2.8 dB/hop. By enforcing a register-aware slope threshold across 2 consecutive hops, v2.0 Apex detects staccato releases in ~10.6 ms while never cutting off high-treble sustains prematurely:

ΔL_hop(v) = L_voice(t − 1) − L_voice(t) DamperDropThreshold(m) = 1.55 dB/hop (for m < 88), 3.20 dB/hop (for m ≥ 88)

6. Engine v1.2 Reference Implementation Code (Preserved from v1.2 Specification)

For complete historical continuity with the v1.2 specification, the original v1.2 Cascade.h and Calibration.h implementations are preserved below, followed in Section 7 by the complete v2.0 Apex Production Architecture.

shared/Cascade.h (v1.2 Reference Architecture)

// shared/Cascade.h (v1.2 Production Architecture Reference) #pragma once #include <algorithm> #include <array> #include <cmath> #include <cstdint> #include <vector> namespace piano { struct NoteEvent { enum Type : uint8_t { NoteOn = 1, NoteOff = 0 } type; int midi; float cents; float velocity; double tMs; }; struct Config { int sampleRate = 48000; int hopSize = 256; // 5.333 ms per hop @ 48 kHz float gateRmsDb = -52.0f; // Calibrated noise floor threshold float onsetFluxRatio = 1.45f; // Sensitive onset ratio for soft 5% touches float goertzelVerifyRatio = 0.08f; // Lowered for soft strike capture int confirmHops = 2; // 2-hop debounce bool strictCalibrationFilter = true; }; struct PianoProfile { bool isCalibrated = false; int minCalibratedMidi = 21; int maxCalibratedMidi = 108; float a4ReferenceHz = 440.0f; float ambientNoiseDb = -56.0f; float calibratedGateDb = -50.0f; float inputGainDb = 0.0f; float inputGainLinear = 1.0f; float calibratedPitchWindowCents = 22.0f; float minCombEnergyRatio = 0.14f; float minTimbreSimilarity = 0.72f; std::array<float, 128> keyFreqHz{}; std::array<float, 128> inharmonicityB{}; std::array<float, 128> keySoftDb{}; // 5%-15% soft dynamic baseline per key std::array<float, 128> keyLoudDb{}; // 85%-100% loud dynamic ceiling per key std::array<float, 8> softPartialWeights{1.0f, 0.25f, 0.08f, 0.03f, 0.01f, 0.00f, 0.00f, 0.00f}; std::array<float, 8> loudPartialWeights{1.0f, 0.75f, 0.52f, 0.36f, 0.24f, 0.16f, 0.10f, 0.06f}; }; class Goertzel { public: static float magnitudeSquared(const float* x, int N, float fHz, int sr) noexcept { const float omega = 2.0f * static_cast<float>(M_PI) * fHz / static_cast<float>(sr); const float coeff = 2.0f * std::cos(omega); float s0 = 0.0f, s1 = 0.0f, s2 = 0.0f; for (int n = 0; n < N; ++n) { s0 = x[n] + coeff * s1 - s2; s2 = s1; s1 = s0; } return s1 * s1 + s2 * s2 - coeff * s1 * s2; } }; struct ActiveVoice { int midi = -1; float fundamentalHz = 0.0f; float inharmonicityB = 0.00035f; float peakDb = -60.0f; float lastDb = -60.0f; float lastCents = 0.0f; int activeHops = 0; bool inDecay = false; }; class Cascade { public: static constexpr int kMaxPolyphony = 6; Cascade() noexcept { for (int m = 0; m < 128; ++m) { profile_.keyFreqHz[m] = 440.0f * std::pow(2.0f, (m - 69) / 12.0f); profile_.inharmonicityB[m] = 0.00035f; profile_.keySoftDb[m] = -42.0f; profile_.keyLoudDb[m] = -10.0f; } for (int v = 0; v < kMaxPolyphony; ++v) voices_[v].midi = -1; } void setConfig(const Config& cfg) noexcept { config_ = cfg; } void setProfile(const PianoProfile& prof) noexcept { profile_ = prof; } const PianoProfile& profile() const noexcept { return profile_; } void setExpectedNotes(const std::vector<int>& expected) noexcept { expectedNotes_ = expected; } void processHop(const float* window4096Raw, const float* hop256Raw, double currentTimeMs, std::vector<NoteEvent>& outEvents) noexcept { const float gain = profile_.isCalibrated ? profile_.inputGainLinear : 1.0f; float sumSq = 0.0f; for (int i = 0; i < 256; ++i) { gainHop256_[i] = hop256Raw[i] * gain; sumSq += gainHop256_[i] * gainHop256_[i]; if (std::fabs(hop256Raw[i]) >= 0.985f) ++clipCount_; } for (int i = 0; i < 4096; ++i) { gainWindow4096_[i] = window4096Raw[i] * gain; } const float rmsLinear = std::sqrt(sumSq / 256.0f + 1e-12f); const float rmsDb = 20.0f * std::log10(rmsLinear); rmsHistory_[rmsHistIdx_] = rmsDb; const int oldIdx = (rmsHistIdx_ + 1) % 4; const float fourHopDeltaDb = rmsDb - rmsHistory_[oldIdx]; rmsHistIdx_ = (rmsHistIdx_ + 1) % 4; const float gateDb = profile_.isCalibrated ? profile_.calibratedGateDb : config_.gateRmsDb; for (int v = 0; v < kMaxPolyphony; ++v) { if (voices_[v].midi < 0) continue; ActiveVoice& voice = voices_[v]; ++voice.activeHops; const int N = (voice.midi < 48) ? 4096 : 1024; const float* buf = (voice.midi < 48) ? gainWindow4096_ : (gainWindow4096_ + 3072); const float e1 = Goertzel::magnitudeSquared(buf, N, voice.fundamentalHz, config_.sampleRate); const float currentVoiceDb = 10.0f * std::log10((e1 / (0.5f * N)) + 1e-12f); const float decaySlope = voice.lastDb - currentVoiceDb; voice.lastDb = currentVoiceDb; if (voice.activeHops >= 4 && (decaySlope > 1.6f ¦¦ currentVoiceDb < gateDb - 4.0f)) { outEvents.push_back({NoteEvent::NoteOff, voice.midi, 0.0f, 0.0f, currentTimeMs}); voice.midi = -1; continue; } if (currentVoiceDb < voice.peakDb - 4.5f) voice.inDecay = true; voice.peakDb = std::max(voice.peakDb, currentVoiceDb); } if (rmsDb < gateDb && activeVoiceCount() == 0) { confirmCount_ = 0; return; } int bestMidi = -1; float bestSalience = 0.0f, bestCombEnergy = 0.0f, bestOvertoneRatio = 0.0f, bestNormIntensity = 0.5f; const bool hasExpected = !expectedNotes_.empty(); const int startMidi = profile_.isCalibrated ? profile_.minCalibratedMidi : 21; const int endMidi = profile_.isCalibrated ? profile_.maxCalibratedMidi : 108; for (int m = startMidi; m <= endMidi; ++m) { if (hasExpected && std::find(expectedNotes_.begin(), expectedNotes_.end(), m) == expectedNotes_.end()) continue; const float f0 = getKeyFreq(m); const float B = profile_.inharmonicityB[m]; const int N = (m < 48) ? 4096 : 1024; const float* buf = (m < 48) ? gainWindow4096_ : (gainWindow4096_ + 3072); const float e1 = Goertzel::magnitudeSquared(buf, N, f0, config_.sampleRate); const float noteDb = 10.0f * std::log10((e1 / (0.5f * N)) + 1e-12f); if (noteDb < profile_.keySoftDb[m] - 5.0f && noteDb < gateDb + 2.0f) continue; const float normI = std::clamp((noteDb - profile_.keySoftDb[m]) / std::max(6.0f, profile_.keyLoudDb[m] - profile_.keySoftDb[m]), 0.0f, 1.0f); std::array<float, 6> ePartials{}, aPartials{}; float combEnergy = 0.0f; for (int h = 1; h <= 6; ++h) { const float fh = h * f0 * std::sqrt(1.0f + B * static_cast<float>(h * h)); if (fh >= config_.sampleRate * 0.46f) break; float eh = Goertzel::magnitudeSquared(buf, N, fh, config_.sampleRate); for (int v = 0; v < kMaxPolyphony; ++v) { if (voices_[v].midi < 0 ¦¦ voices_[v].midi == m) continue; const float vf0 = voices_[v].fundamentalHz; const float vB = voices_[v].inharmonicityB; for (int vh = 1; vh <= 6; ++vh) { const float vfh = vh * vf0 * std::sqrt(1.0f + vB * static_cast<float>(vh * vh)); if (std::fabs(fh - vfh) < (config_.sampleRate / static_cast<float>(N))) { eh = std::max(0.0f, eh - 0.75f * eh); } } } ePartials[h - 1] = eh; aPartials[h - 1] = std::sqrt(std::max(0.0f, eh)); combEnergy += eh; } const float overtoneSum = ePartials[1] + ePartials[2] + ePartials[3]; const float overtoneRatio = overtoneSum / (ePartials[0] + 1e-9f); float dot = 0.0f, normA = 0.0f, normW = 0.0f; for (int h = 0; h < 6; ++h) { const float wTarget = (1.0f - normI) * profile_.softPartialWeights[h] + normI * profile_.loudPartialWeights[h]; dot += aPartials[h] * wTarget; normA += aPartials[h] * aPartials[h]; normW += wTarget * wTarget; } const float cosineSim = dot / (std::sqrt(normA * normW) + 1e-9f); if (profile_.isCalibrated && config_.strictCalibrationFilter && m >= 36 && m <= 88) { if (cosineSim < profile_.minTimbreSimilarity) continue; } const int activeIdx = findVoice(m); if (activeIdx >= 0 && (!voices_[activeIdx].inDecay ¦¦ fourHopDeltaDb < 3.5f)) continue; const float salience = ePartials[0] * 1.0f + ePartials[1] * 0.7f + ePartials[2] * 0.4f; if (salience > bestSalience) { bestSalience = salience; bestMidi = m; bestCombEnergy = combEnergy; bestOvertoneRatio = overtoneRatio; bestNormIntensity = normI; } } if (bestMidi < 0) { confirmCount_ = 0; return; } const int bestN = (bestMidi < 48) ? 4096 : 1024; const float* bestBuf = (bestMidi < 48) ? gainWindow4096_ : (gainWindow4096_ + 3072); float windowEnergy = 0.0f; for (int i = 0; i < bestN; ++i) windowEnergy += bestBuf[i] * bestBuf[i]; const float normCombRatio = (bestCombEnergy / (0.5f * static_cast<float>(bestN))) / (windowEnergy + 1e-9f); if (normCombRatio < profile_.minCombEnergyRatio) { confirmCount_ = 0; return; } const float bestCents = estimateFineCents(bestBuf, bestN, getKeyFreq(bestMidi)); if (profile_.isCalibrated && config_.strictCalibrationFilter && std::fabs(bestCents) > profile_.calibratedPitchWindowCents) { confirmCount_ = 0; return; } if (bestMidi == candidateMidi_ && std::fabs(bestCents - candidateCents_) < 18.0f) { if (++confirmCount_ >= config_.confirmHops) { const float brightnessRatio = std::clamp(bestOvertoneRatio / 1.8f, 0.0f, 1.0f); const float finalVelocity = 0.05f + 0.95f * (0.65f * bestNormIntensity + 0.35f * brightnessRatio); int voiceSlot = findVoice(bestMidi); if (voiceSlot < 0) voiceSlot = findFreeVoiceSlot(); if (voiceSlot >= 0) { voices_[voiceSlot] = {bestMidi, getKeyFreq(bestMidi), profile_.inharmonicityB[bestMidi], rmsDb, rmsDb, bestCents, 0, false}; } confirmCount_ = 0; outEvents.push_back({NoteEvent::NoteOn, bestMidi, bestCents, finalVelocity, currentTimeMs}); } } else { candidateMidi_ = bestMidi; candidateCents_ = bestCents; confirmCount_ = 1; } } private: int findVoice(int midi) const noexcept { for (int v = 0; v < kMaxPolyphony; ++v) if (voices_[v].midi == midi) return v; return -1; } int findFreeVoiceSlot() noexcept { for (int v = 0; v < kMaxPolyphony; ++v) if (voices_[v].midi < 0) return v; int oldestIdx = 0, maxHops = -1; for (int v = 0; v < kMaxPolyphony; ++v) { if (voices_[v].activeHops > maxHops) { maxHops = voices_[v].activeHops; oldestIdx = v; } } return oldestIdx; } int activeVoiceCount() const noexcept { int c = 0; for (int v = 0; v < kMaxPolyphony; ++v) if (voices_[v].midi >= 0) ++c; return c; } float getKeyFreq(int midi) const noexcept { if (midi >= 0 && midi < 128 && profile_.keyFreqHz[midi] > 20.0f) return profile_.keyFreqHz[midi]; return 440.0f * std::pow(2.0f, (midi - 69) / 12.0f); } float estimateFineCents(const float* buf, int N, float centerHz) const noexcept { const float magCenter = Goertzel::magnitudeSquared(buf, N, centerHz, config_.sampleRate); const float hzMinus = centerHz * std::pow(2.0f, -15.0f / 1200.0f); const float hzPlus = centerHz * std::pow(2.0f, +15.0f / 1200.0f); const float magMinus = Goertzel::magnitudeSquared(buf, N, hzMinus, config_.sampleRate); const float magPlus = Goertzel::magnitudeSquared(buf, N, hzPlus, config_.sampleRate); const float denom = magMinus - 2.0f * magCenter + magPlus; if (std::fabs(denom) < 1e-9f) return 0.0f; return std::clamp(0.5f * (magMinus - magPlus) / denom * 15.0f, -35.0f, 35.0f); } Config config_{}; PianoProfile profile_{}; std::vector<int> expectedNotes_{}; std::array<ActiveVoice, kMaxPolyphony> voices_{}; float gainHop256_[256]{}; float gainWindow4096_[4096]{}; std::array<float, 4> rmsHistory_{-60.0f, -60.0f, -60.0f, -60.0f}; int rmsHistIdx_ = 0, candidateMidi_ = -1, confirmCount_ = 0; float candidateCents_ = 0.0f; uint32_t clipCount_ = 0; }; } // namespace piano

7. Complete Production Implementation: Engine v2.0 Apex Architecture

v2.0 Apex Step 1: Zero-Drift Cache-Aligned SPSC Ring Buffer (shared/SampleRing.h)

shared/SampleRing.h (v2.0 Apex Production Architecture)

// shared/SampleRing.h (v2.0 Apex Production Architecture) #pragma once #include <atomic> #include <cstddef> #include <cstdint> #include <cstring> #include <algorithm> namespace piano { template <std::size_t CapacityPow2 = 16384> class SampleRing { static_assert((CapacityPow2 & (CapacityPow2 - 1)) == 0, "CapacityPow2 must be a power of two"); static constexpr std::size_t kMask = CapacityPow2 - 1; public: SampleRing() = default; std::size_t push(const float* data, std::size_t count) noexcept { const std::size_t tail = tail_.load(std::memory_order_relaxed); const std::size_t head = head_.load(std::memory_order_acquire); const std::size_t avail = CapacityPow2 - (tail - head); const std::size_t toWrite = std::min(count, avail); for (std::size_t i = 0; i < toWrite; ++i) { buffer_[(tail + i) & kMask] = data[i]; } tail_.store(tail + toWrite, std::memory_order_release); return toWrite; } std::size_t available() const noexcept { const std::size_t tail = tail_.load(std::memory_order_acquire); const std::size_t head = head_.load(std::memory_order_relaxed); return tail - head; } // Fast-forwards read cursor if OS audio thread lag generates > 4 hops of backlog bool dropStaleBacklog(std::size_t hopSize = 256, std::size_t maxBacklogHops = 4) noexcept { const std::size_t tail = tail_.load(std::memory_order_acquire); const std::size_t head = head_.load(std::memory_order_relaxed); if (tail - head > hopSize * maxBacklogHops) { head_.store(tail - hopSize, std::memory_order_release); return true; } return false; } // Copies windowSize samples ending exactly at the boundary of the current hop void peekWindow(float* dst, std::size_t windowSize, std::size_t hopSize = 256) const noexcept { const std::size_t endCursor = head_.load(std::memory_order_relaxed) + hopSize; const std::size_t startCursor = endCursor - windowSize; for (std::size_t i = 0; i < windowSize; ++i) { dst[i] = buffer_[(startCursor + i) & kMask]; } } void consume(std::size_t hopSize = 256) noexcept { const std::size_t head = head_.load(std::memory_order_relaxed); head_.store(head + hopSize, std::memory_order_release); } private: alignas(64) std::atomic<std::size_t> head_{4096}; alignas(64) std::atomic<std::size_t> tail_{4096}; alignas(64) float buffer_[CapacityPow2]{}; }; } // namespace piano

v2.0 Apex Step 2: 3-Tier Multi-Rate, 4x-SIMD Goertzel & 10-Voice Chord Peeling Core (shared/Cascade.h)

shared/Cascade.h (v2.0 Apex Production Architecture)

// shared/Cascade.h (v2.0 Apex Production Architecture) #pragma once #include <algorithm> #include <array> #include <atomic> #include <cmath> #include <cstdint> #include <vector> namespace piano { struct NoteEvent { enum Type : uint8_t { NoteOn = 1, NoteOff = 0 } type; int midi; float cents; float velocity; double tMs; }; struct Config { int sampleRate = 48000; int hopSize = 256; // 5.333 ms per hop @ 48 kHz float gateRmsDb = -52.0f; // Uncalibrated fallback noise floor float onsetFluxRatio = 1.45f; // Sensitive onset ratio for 5% touches float goertzelVerifyRatio = 0.08f; // Soft-strike harmonic verification int confirmHops = 2; // 2-hop confirmation (10.66 ms) bool strictCalibrationFilter = true; bool autoAdaptTuning = true; // Online EMA tuning drift tracker }; struct PianoProfile { bool isCalibrated = false; int minCalibratedMidi = 21; int maxCalibratedMidi = 108; float a4ReferenceHz = 440.0f; float ambientNoiseDb = -56.0f; float calibratedGateDb = -50.0f; float inputGainDb = 0.0f; float inputGainLinear = 1.0f; float calibratedPitchWindowCents = 24.0f; float minCombEnergyRatio = 0.12f; float minTimbreSimilarity = 0.70f; std::array<float, 128> keyFreqHz{}; std::array<float, 128> inharmonicityB{}; std::array<float, 128> keySoftDb{}; // Post-gain 5%-15% soft dynamic baseline per key std::array<float, 128> keyLoudDb{}; // Post-gain 85%-100% loud dynamic ceiling per key std::array<float, 8> softPartialWeights{1.0f, 0.25f, 0.08f, 0.03f, 0.01f, 0.00f, 0.00f, 0.00f}; std::array<float, 8> loudPartialWeights{1.0f, 0.75f, 0.52f, 0.36f, 0.24f, 0.16f, 0.10f, 0.06f}; }; class Goertzel { public: static float magnitudeSquared(const float* x, int N, float fHz, int sr) noexcept { const float omega = 2.0f * static_cast<float>(M_PI) * fHz / static_cast<float>(sr); const float coeff = 2.0f * std::cos(omega); return magnitudeSquaredPrecomp(x, N, coeff); } static inline float magnitudeSquaredPrecomp(const float* x, int N, float coeff) noexcept { float s0 = 0.0f, s1 = 0.0f, s2 = 0.0f; for (int n = 0; n < N; ++n) { s0 = x[n] + coeff * s1 - s2; s2 = s1; s1 = s0; } return s1 * s1 + s2 * s2 - coeff * s1 * s2; } // 4-Way SIMD Vectorized Goertzel: Evaluates 4 partials simultaneously in a single pass static inline void magnitudeSquared4x(const float* __restrict x, int N, const float* __restrict coeffs4, float* __restrict outMagSq4) noexcept { alignas(16) float s1[4] = {0.0f, 0.0f, 0.0f, 0.0f}; alignas(16) float s2[4] = {0.0f, 0.0f, 0.0f, 0.0f}; alignas(16) const float c[4] = {coeffs4[0], coeffs4[1], coeffs4[2], coeffs4[3]}; for (int n = 0; n < N; ++n) { const float xn = x[n]; for (int k = 0; k < 4; ++k) { const float s0 = xn + c[k] * s1[k] - s2[k]; s2[k] = s1[k]; s1[k] = s0; } } for (int k = 0; k < 4; ++k) { outMagSq4[k] = std::max(0.0f, s1[k] * s1[k] + s2[k] * s2[k] - c[k] * s1[k] * s2[k]); } } // Converts Hann-windowed Goertzel magSq to exact time-domain mean-square power (A^2 / 2) // Coherent gain of Hann window is 0.5 -> (N * 0.5)^2 / 2 = 0.125 * N^2 static inline float toMeanSquarePower(float magSq, int N) noexcept { const float normFactor = 0.125f * static_cast<float>(N) * static_cast<float>(N); return magSq / normFactor; } }; struct ActiveVoice { int midi = -1; float fundamentalHz = 0.0f; float inharmonicityB = 0.00035f; float peakDb = -60.0f; float lastDb = -60.0f; float lastCents = 0.0f; float normIntensity = 0.5f; int activeHops = 0; int damperDropHops = 0; bool inDecay = false; std::array<float, 6> partialPower{}; // True physical mean-square power of partials h=1..6 }; class Cascade { public: static constexpr int kMaxPolyphony = 10; // 10-voice polyphony for 2-hand + pedal play static constexpr int kMaxSimultaneousOnsets = 4; // Max chord notes confirmed in same hop Cascade() noexcept { initWindows(); PianoProfile defaultProf{}; for (int m = 0; m < 128; ++m) { defaultProf.keyFreqHz[m] = 440.0f * std::pow(2.0f, (m - 69) / 12.0f); defaultProf.inharmonicityB[m] = (m < 48) ? 0.00035f * std::exp(0.035f * (48 - m)) : 0.00035f * std::exp(0.042f * (m - 48)); defaultProf.keySoftDb[m] = -44.0f; defaultProf.keyLoudDb[m] = -10.0f; } setProfile(defaultProf); for (int v = 0; v < kMaxPolyphony; ++v) voices_[v].midi = -1; } void setConfig(const Config& cfg) noexcept { config_ = cfg; rebuildLookupTables(profiles_[activeProfileIdx_.load(std::memory_order_relaxed)]); } // Thread-safe double-buffered profile swap (Safe to call from JS/TurboModule thread) void setProfile(const PianoProfile& prof) noexcept { const uint32_t currentIdx = activeProfileIdx_.load(std::memory_order_relaxed); const uint32_t nextIdx = 1U - currentIdx; profiles_[nextIdx] = prof; rebuildLookupTables(profiles_[nextIdx], nextIdx); activeProfileIdx_.store(nextIdx, std::memory_order_release); } const PianoProfile& profile() const noexcept { return profiles_[activeProfileIdx_.load(std::memory_order_acquire)]; } // Wait-free O(1) 128-bit atomic bitmask for expected notes (Zero heap allocation on audio thread) void setExpectedNotes(const std::vector<int>& expected) noexcept { uint64_t low = 0ULL; uint64_t high = 0ULL; for (int m : expected) { if (m >= 0 && m < 64) low ¦= (1ULL << m); else if (m >= 64 && m < 128) high ¦= (1ULL << (m - 64)); } expectedMaskLow_.store(low, std::memory_order_release); expectedMaskHigh_.store(high, std::memory_order_release); } void processHop(const float* window4096Raw, const float* hop256Raw, double currentTimeMs, std::vector<NoteEvent>& outEvents) noexcept { const uint32_t profIdx = activeProfileIdx_.load(std::memory_order_acquire); const PianoProfile& prof = profiles_[profIdx]; // Stage 1: Calibrated Input Gain, Clipping Guard & Hann-Windowed Multi-Rate Buffers const float gain = prof.isCalibrated ? prof.inputGainLinear : 1.0f; float sumSq = 0.0f; int hopClips = 0; for (int i = 0; i < 256; ++i) { gainHop256_[i] = hop256Raw[i] * gain; sumSq += gainHop256_[i] * gainHop256_[i]; if (std::fabs(hop256Raw[i]) >= 0.985f) ++hopClips; } clipCount_ += hopClips; const float rmsMeanSq = sumSq / 256.0f + 1e-12f; const float rmsDb = 10.0f * std::log10(rmsMeanSq); // 4-hop cumulative onset tracker + instantaneous attack transient detector rmsHistory_[rmsHistIdx_] = rmsDb; const int oldIdx = (rmsHistIdx_ + 1) & 3; const float fourHopDeltaDb = rmsDb - rmsHistory_[oldIdx]; const float oneHopDeltaDb = rmsDb - prevHopRmsDb_; prevHopRmsDb_ = rmsDb; rmsHistIdx_ = (rmsHistIdx_ + 1) & 3; if (oneHopDeltaDb > 6.5f) transientHopsRemaining_ = 2; else if (transientHopsRemaining_ > 0) --transientHopsRemaining_; const float gateDb = prof.isCalibrated ? prof.calibratedGateDb : config_.gateRmsDb; if (rmsDb < gateDb && activeVoiceCount() == 0) { clearConfirmations(); return; } // Prepare 3-tier Hann-windowed buffers & normalized window powers once per hop float energy4096 = 0.0f, energy2048 = 0.0f, energy1024 = 0.0f; for (int i = 0; i < 4096; ++i) { const float s = window4096Raw[i] * gain; gainWindow4096_[i] = s; win4096_[i] = s * hann4096_[i]; energy4096 += s * s; } const float* raw2048 = gainWindow4096_ + 2048; for (int i = 0; i < 2048; ++i) { win2048_[i] = raw2048[i] * hann2048_[i]; energy2048 += raw2048[i] * raw2048[i]; } const float* raw1024 = gainWindow4096_ + 3072; for (int i = 0; i < 1024; ++i) { win1024_[i] = raw1024[i] * hann1024_[i]; energy1024 += raw1024[i] * raw1024[i]; } const float meanSqWin4096 = energy4096 / 4096.0f + 1e-12f; const float meanSqWin2048 = energy2048 / 2048.0f + 1e-12f; const float meanSqWin1024 = energy1024 / 1024.0f + 1e-12f; // Stage 2: Polyphonic Voice Maintenance, Physical Partial Tracking & Damper Articulation for (int v = 0; v < kMaxPolyphony; ++v) { if (voices_[v].midi < 0) continue; ActiveVoice& voice = voices_[v]; ++voice.activeHops; const int m = voice.midi; const int N = getWindowSizeForMidi(m); const float* wBuf = getWindowedBufferForMidi(m); alignas(16) float magSq4[4]{}; Goertzel::magnitudeSquared4x(wBuf, N, coeffTable_[profIdx][m].data(), magSq4); for (int h = 0; h < 4; ++h) { voice.partialPower[h] = Goertzel::toMeanSquarePower(magSq4[h], N); } for (int h = 4; h < 6; ++h) { const float c = coeffTable_[profIdx][m][h]; voice.partialPower[h] = (c > -2.0f) ? Goertzel::toMeanSquarePower(Goertzel::magnitudeSquaredPrecomp(wBuf, N, c), N) : 0.0f; } const float trackedPower = (m < 36) ? std::max(voice.partialPower[0], 0.7f * (voice.partialPower[1] + voice.partialPower[2])) : voice.partialPower[0]; const float currentVoiceDb = 10.0f * std::log10(trackedPower + 1e-12f); const float decaySlope = voice.lastDb - currentVoiceDb; voice.lastDb = currentVoiceDb; // Register-aware damper-drop slope: Undamped high treble (m >= 88) decays faster naturally const float damperSlopeThreshold = (m >= 88) ? 3.2f : 1.55f; if (decaySlope > damperSlopeThreshold) ++voice.damperDropHops; else voice.damperDropHops = 0; if (voice.activeHops >= 4 && (voice.damperDropHops >= 2 ¦¦ currentVoiceDb < gateDb - 4.0f)) { outEvents.push_back({NoteEvent::NoteOff, voice.midi, 0.0f, 0.0f, currentTimeMs}); voice.midi = -1; continue; } if (currentVoiceDb < voice.peakDb - 4.0f) { voice.inDecay = true; } voice.peakDb = std::max(voice.peakDb, currentVoiceDb); } if (rmsDb < gateDb) { clearConfirmations(); return; } // Stage 3: Pre-Scan Candidate Partial Power Matrix across 88 Keys const uint64_t expLow = expectedMaskLow_.load(std::memory_order_acquire); const uint64_t expHigh = expectedMaskHigh_.load(std::memory_order_acquire); const bool hasExpected = (expLow != 0ULL) ¦¦ (expHigh != 0ULL); const int startMidi = prof.isCalibrated ? prof.minCalibratedMidi : 21; const int endMidi = prof.isCalibrated ? prof.maxCalibratedMidi : 108; std::array<bool, 128> candActive{}; for (int m = startMidi; m <= endMidi; ++m) { if (hasExpected && !isNoteExpected(m, expLow, expHigh)) continue; const int activeIdx = findVoice(m); if (activeIdx >= 0 && (!voices_[activeIdx].inDecay ¦¦ fourHopDeltaDb < 3.2f)) { continue; } const int N = getWindowSizeForMidi(m); const float* wBuf = getWindowedBufferForMidi(m); alignas(16) float magSq4[4]{}; Goertzel::magnitudeSquared4x(wBuf, N, coeffTable_[profIdx][m].data(), magSq4); for (int h = 0; h < 4; ++h) { candPartialPower_[m][h] = Goertzel::toMeanSquarePower(magSq4[h], N); } const float gatePower = (m < 36) ? std::max(candPartialPower_[m][0], 0.7f * (candPartialPower_[m][1] + candPartialPower_[m][2])) : candPartialPower_[m][0]; const float noteDb = 10.0f * std::log10(gatePower + 1e-12f); if (noteDb < prof.keySoftDb[m] - 5.0f && noteDb < gateDb + 1.5f) continue; for (int h = 4; h < 6; ++h) { const float c = coeffTable_[profIdx][m][h]; candPartialPower_[m][h] = (c > -2.0f) ? Goertzel::toMeanSquarePower(Goertzel::magnitudeSquaredPrecomp(wBuf, N, c), N) : 0.0f; } // Subtract physical partial power of already-active sustained voices for (int h = 0; h < 6; ++h) { const float fh = partialFreqTable_[profIdx][m][h]; if (fh <= 0.0f) break; const float binBw = static_cast<float>(config_.sampleRate) / static_cast<float>(N); for (int v = 0; v < kMaxPolyphony; ++v) { if (voices_[v].midi < 0 ¦¦ voices_[v].midi == m) continue; const int vm = voices_[v].midi; for (int vh = 0; vh < 6; ++vh) { const float vfh = partialFreqTable_[profIdx][vm][vh]; const float df = std::fabs(fh - vfh); if (df < binBw) { const float overlap = 1.0f - 0.5f * (df / binBw); candPartialPower_[m][h] = std::max( 0.0f, candPartialPower_[m][h] - 1.15f * overlap * voices_[v].partialPower[vh]); } } } } candActive[m] = true; } // Stage 4: Intra-Hop Iterative Klapuri Peeling for Simultaneous Block Chords std::array<bool, 128> confirmedInHop{}; for (int peel = 0; peel < kMaxSimultaneousOnsets; ++peel) { int bestMidi = -1; float bestSalience = 0.0f; float bestCombPower = 0.0f; float bestNormIntensity = 0.5f; for (int m = startMidi; m <= endMidi; ++m) { if (!candActive[m]) continue; const float p1 = candPartialPower_[m][0]; const float p2 = candPartialPower_[m][1]; const float p3 = candPartialPower_[m][2]; const float p4 = candPartialPower_[m][3]; const float p5 = candPartialPower_[m][4]; const float p6 = candPartialPower_[m][5]; const float effectiveFundPower = (m < 36) ? std::max(p1, 0.65f * (p2 + p3)) : p1; const float noteDb = 10.0f * std::log10(effectiveFundPower + 1e-12f); if (noteDb < prof.keySoftDb[m] - 5.0f && noteDb < gateDb + 1.5f) continue; const float normI = std::clamp( (noteDb - prof.keySoftDb[m]) / std::max(6.0f, prof.keyLoudDb[m] - prof.keySoftDb[m]), 0.0f, 1.0f); const float combPower = p1 + p2 + p3 + p4 + p5 + p6; // Dual-Profile Timbre Cosine Similarity Gate float dot = 0.0f, normA = 0.0f, normW = 0.0f; for (int h = 0; h < 6; ++h) { const float ampH = std::sqrt(candPartialPower_[m][h]); const float wTarget = (1.0f - normI) * prof.softPartialWeights[h] + normI * prof.loudPartialWeights[h]; dot += ampH * wTarget; normA += ampH * ampH; normW += wTarget * wTarget; } const float cosineSim = dot / (std::sqrt(normA * normW) + 1e-9f); if (prof.isCalibrated && config_.strictCalibrationFilter && m >= 36 && m <= 88) { if (cosineSim < prof.minTimbreSimilarity) continue; } // Odd-Harmonic Disambiguation Bonus (Prevents A0 -> A1 bass octave errors) float oddBonus = 1.0f; if (m < 48) { const float oddRatio = (p3 + p5) / (p2 + p4 + 1e-12f); if (oddRatio > 0.12f) oddBonus = 1.35f; } const float salience = oddBonus * (p1 * 1.0f + p2 * 0.75f + p3 * 0.50f + p4 * 0.25f); if (salience > bestSalience) { bestSalience = salience; bestMidi = m; bestCombPower = combPower; bestNormIntensity = normI; } } if (bestMidi < 0) break; const float meanSqWin = getMeanSqWindowForMidi(bestMidi, meanSqWin4096, meanSqWin2048, meanSqWin1024); const float normCombRatio = bestCombPower / meanSqWin; const float requiredCombRatio = (peel == 0) ? prof.minCombEnergyRatio : (prof.minCombEnergyRatio * 0.45f); if (normCombRatio < requiredCombRatio) { candActive[bestMidi] = false; continue; } const int bestN = getWindowSizeForMidi(bestMidi); const float* bestWBuf = getWindowedBufferForMidi(bestMidi); const float bestCents = estimateFineCents(bestWBuf, bestN, getKeyFreq(prof, bestMidi)); const float maxAllowedCents = (transientHopsRemaining_ > 0) ? (prof.calibratedPitchWindowCents + 10.0f) : prof.calibratedPitchWindowCents; if (prof.isCalibrated && config_.strictCalibrationFilter && std::fabs(bestCents) > maxAllowedCents) { candActive[bestMidi] = false; continue; } // Subtract winner's stretched harmonics from remaining candidates in this hop for (int m = startMidi; m <= endMidi; ++m) { if (!candActive[m] ¦¦ m == bestMidi) continue; const int N = getWindowSizeForMidi(m); const float binBw = static_cast<float>(config_.sampleRate) / static_cast<float>(N); for (int h = 0; h < 6; ++h) { const float fh = partialFreqTable_[profIdx][m][h]; if (fh <= 0.0f) break; for (int wh = 0; wh < 6; ++wh) { const float wfh = partialFreqTable_[profIdx][bestMidi][wh]; if (std::fabs(fh - wfh) < binBw) { candPartialPower_[m][h] = std::max( 0.0f, candPartialPower_[m][h] - 1.25f * candPartialPower_[bestMidi][wh]); } } } } candActive[bestMidi] = false; confirmedInHop[bestMidi] = true; // Per-Key Confirmation State Machine (Enables simultaneous chord onsets) const float centJitterTol = (transientHopsRemaining_ > 0) ? 26.0f : 18.0f; if (keyConfirmCount_[bestMidi] > 0 && std::fabs(bestCents - keyCandidateCents_[bestMidi]) < centJitterTol) { if (++keyConfirmCount_[bestMidi] >= config_.confirmHops) { assignVoiceSlot(prof, bestMidi, bestCents, bestNormIntensity, candPartialPower_[bestMidi], currentTimeMs, outEvents); keyConfirmCount_[bestMidi] = 0; if (config_.autoAdaptTuning && peel == 0 && activeVoiceCount() == 1 && bestMidi >= 48 && bestMidi <= 76 && std::fabs(bestCents) < 15.0f) { emaTuningCents_ = 0.92f * emaTuningCents_ + 0.08f * bestCents; } } } else { keyConfirmCount_[bestMidi] = 1; keyCandidateCents_[bestMidi] = bestCents; } } for (int m = startMidi; m <= endMidi; ++m) { if (!confirmedInHop[m]) keyConfirmCount_[m] = 0; } } float estimatedTuningDriftCents() const noexcept { return emaTuningCents_; } uint32_t clipCount() const noexcept { return clipCount_; } private: void initWindows() noexcept { for (int i = 0; i < 4096; ++i) { hann4096_[i] = 0.5f * (1.0f - std::cos(2.0f * static_cast<float>(M_PI) * i / 4095.0f)); } for (int i = 0; i < 2048; ++i) { hann2048_[i] = 0.5f * (1.0f - std::cos(2.0f * static_cast<float>(M_PI) * i / 2047.0f)); } for (int i = 0; i < 1024; ++i) { hann1024_[i] = 0.5f * (1.0f - std::cos(2.0f * static_cast<float>(M_PI) * i / 1023.0f)); } } void rebuildLookupTables(const PianoProfile& prof, uint32_t idx = 0) noexcept { const float nyquistGuard = config_.sampleRate * 0.46f; for (int m = 0; m < 128; ++m) { const float f0 = getKeyFreq(prof, m); const float B = prof.inharmonicityB[m]; for (int h = 1; h <= 6; ++h) { const float fh = h * f0 * std::sqrt(1.0f + B * static_cast<float>(h * h)); if (fh < nyquistGuard) { partialFreqTable_[idx][m][h - 1] = fh; const float omega = 2.0f * static_cast<float>(M_PI) * fh / static_cast<float>(config_.sampleRate); coeffTable_[idx][m][h - 1] = 2.0f * std::cos(omega); } else { partialFreqTable_[idx][m][h - 1] = 0.0f; coeffTable_[idx][m][h - 1] = -4.0f; } } } } static inline int getWindowSizeForMidi(int m) noexcept { if (m < 42) return 4096; // A0 - F2: 85.3 ms lookback if (m < 66) return 2048; // F#2 - F#4: 42.6 ms lookback return 1024; // G4 - C8: 21.3 ms lookback } inline const float* getWindowedBufferForMidi(int m) const noexcept { if (m < 42) return win4096_; if (m < 66) return win2048_; return win1024_; } static inline float getMeanSqWindowForMidi(int m, float ms4096, float ms2048, float ms1024) noexcept { if (m < 42) return ms4096; if (m < 66) return ms2048; return ms1024; } static inline bool isNoteExpected(int m, uint64_t low, uint64_t high) noexcept { if (m < 64) return ((low >> m) & 1ULL) != 0ULL; return ((high >> (m - 64)) & 1ULL) != 0ULL; } void clearConfirmations() noexcept { keyConfirmCount_.fill(0); } void assignVoiceSlot(const PianoProfile& prof, int midi, float cents, float normI, const std::array<float, 6>& partialPowers, double currentTimeMs, std::vector<NoteEvent>& outEvents) noexcept { int slot = findVoice(midi); if (slot >= 0) { outEvents.push_back({NoteEvent::NoteOff, midi, 0.0f, 0.0f, currentTimeMs}); } else { slot = findFreeVoiceSlot(currentTimeMs, outEvents); } const float trackedP = (midi < 36) ? std::max(partialPowers[0], 0.7f * (partialPowers[1] + partialPowers[2])) : partialPowers[0]; const float initVoiceDb = 10.0f * std::log10(trackedP + 1e-12f); voices_[slot].midi = midi; voices_[slot].fundamentalHz = getKeyFreq(prof, midi); voices_[slot].inharmonicityB = prof.inharmonicityB[midi]; voices_[slot].peakDb = initVoiceDb; voices_[slot].lastDb = initVoiceDb; voices_[slot].lastCents = cents; voices_[slot].normIntensity = normI; voices_[slot].activeHops = 0; voices_[slot].damperDropHops = 0; voices_[slot].inDecay = false; voices_[slot].partialPower = partialPowers; const float brightnessRatio = std::clamp( (partialPowers[1] + partialPowers[2] + partialPowers[3]) / (trackedP * 1.8f + 1e-12f), 0.0f, 1.0f); const float finalVelocity = std::clamp(0.05f + 0.95f * (0.65f * normI + 0.35f * brightnessRatio), 0.05f, 1.0f); outEvents.push_back({NoteEvent::NoteOn, midi, cents, finalVelocity, currentTimeMs}); } int findVoice(int midi) const noexcept { for (int v = 0; v < kMaxPolyphony; ++v) { if (voices_[v].midi == midi) return v; } return -1; } int findFreeVoiceSlot(double currentTimeMs, std::vector<NoteEvent>& outEvents) noexcept { for (int v = 0; v < kMaxPolyphony; ++v) { if (voices_[v].midi < 0) return v; } int oldestIdx = 0, maxHops = -1; for (int v = 0; v < kMaxPolyphony; ++v) { if (voices_[v].activeHops > maxHops) { maxHops = voices_[v].activeHops; oldestIdx = v; } } if (voices_[oldestIdx].midi >= 0) { outEvents.push_back({NoteEvent::NoteOff, voices_[oldestIdx].midi, 0.0f, 0.0f, currentTimeMs}); voices_[oldestIdx].midi = -1; } return oldestIdx; } int activeVoiceCount() const noexcept { int c = 0; for (int v = 0; v < kMaxPolyphony; ++v) if (voices_[v].midi >= 0) ++c; return c; } static float getKeyFreq(const PianoProfile& prof, int midi) noexcept { if (midi >= 0 && midi < 128 && prof.keyFreqHz[midi] > 20.0f) { return prof.keyFreqHz[midi]; } return 440.0f * std::pow(2.0f, (midi - 69) / 12.0f); } float estimateFineCents(const float* buf, int N, float centerHz) const noexcept { const float magCenter = Goertzel::magnitudeSquared(buf, N, centerHz, config_.sampleRate); const float hzMinus = centerHz * 0.991373f; // 2^(-15/1200) const float hzPlus = centerHz * 1.008701f; // 2^(+15/1200) const float magMinus = Goertzel::magnitudeSquared(buf, N, hzMinus, config_.sampleRate); const float magPlus = Goertzel::magnitudeSquared(buf, N, hzPlus, config_.sampleRate); const float denom = magMinus - 2.0f * magCenter + magPlus; if (std::fabs(denom) < 1e-9f) return 0.0f; const float delta = 0.5f * (magMinus - magPlus) / denom; return std::clamp(delta * 15.0f, -35.0f, 35.0f); } Config config_{}; std::array<PianoProfile, 2> profiles_{}; std::atomic<uint32_t> activeProfileIdx_{0}; std::atomic<uint64_t> expectedMaskLow_{0ULL}; std::atomic<uint64_t> expectedMaskHigh_{0ULL}; std::array<std::array<std::array<float, 6>, 128>, 2> coeffTable_{}; std::array<std::array<std::array<float, 6>, 128>, 2> partialFreqTable_{}; std::array<std::array<float, 6>, 128> candPartialPower_{}; std::array<ActiveVoice, kMaxPolyphony> voices_{}; std::array<uint8_t, 128> keyConfirmCount_{}; std::array<float, 128> keyCandidateCents_{}; alignas(64) float hann4096_[4096]{}; alignas(64) float hann2048_[2048]{}; alignas(64) float hann1024_[1024]{}; alignas(64) float gainHop256_[256]{}; alignas(64) float gainWindow4096_[4096]{}; alignas(64) float win4096_[4096]{}; alignas(64) float win2048_[2048]{}; alignas(64) float win1024_[1024]{}; std::array<float, 4> rmsHistory_{-60.0f, -60.0f, -60.0f, -60.0f}; int rmsHistIdx_ = 0; float prevHopRmsDb_ = -60.0f; int transientHopsRemaining_ = 0; float emaTuningCents_ = 0.0f; uint32_t clipCount_ = 0; }; } // namespace piano

v2.0 Apex Step 3: Post-Gain Aligned Two-Strike Calibrator with Serialization (shared/Calibration.h)

shared/Calibration.h (v2.0 Apex Production Architecture)

// shared/Calibration.h (v2.0 Apex Production Architecture) #pragma once #include "Cascade.h" #include <algorithm> #include <array> #include <cmath> #include <cstdint> #include <vector> namespace piano { struct CalibrationProgressEvent { int currentMidi; int keyIndex; int totalKeys; int strikeNumber; // 1 (Soft 5%-20%) or 2 (Firm 75%-100%) int state; // -2=NonPianoNoise, -1=WrongKey, 0=Noise, 1=Wait1, 2=Wait2, 3=KeyDone, 4=AllDone float measuredFreqHz; float centsOffset; float strikeRmsDb; float measuredB; float recommendedGainDb; bool isClipping; }; class PianoCalibrator { public: static constexpr std::array<int, 7> kFastSplitScaleSeq = {24, 36, 48, 60, 69, 72, 84}; static constexpr std::array<int, 16> kFullChromaticSeq = { 24, 36, 48, 60, 61, 62, 63, 64, 65, 66, 67, 68, 69, 70, 71, 72 }; PianoCalibrator() noexcept { for (int i = 0; i < 4096; ++i) { hann4096_[i] = 0.5f * (1.0f - std::cos(2.0f * static_cast<float>(M_PI) * i / 4095.0f)); } } void start(int sampleRate = 48000, bool fullChromaticMode = false) noexcept { sr_ = sampleRate; fullMode_ = fullChromaticMode; totalKeys_ = fullChromaticMode ? 16 : 7; keyIdx_ = 0; strikeIdx_ = 1; state_ = State::MeasureNoise; noiseHops_ = 0; noiseEnergySum_ = 0.0f; rawFirmPeakMax_ = -60.0f; clipDetected_ = false; for (int m = 0; m < 128; ++m) { profile_.keyFreqHz[m] = 440.0f * std::pow(2.0f, (m - 69) / 12.0f); profile_.inharmonicityB[m] = 0.00035f; profile_.keySoftDb[m] = -44.0f; profile_.keyLoudDb[m] = -10.0f; measuredKeyMask_[m] = false; } profile_.softPartialWeights = {1.0f, 0.25f, 0.08f, 0.03f, 0.01f, 0.00f, 0.00f, 0.00f}; profile_.loudPartialWeights = {1.0f, 0.75f, 0.52f, 0.36f, 0.24f, 0.16f, 0.10f, 0.06f}; } bool processWindow(const float* window4096, const float* hop256, CalibrationProgressEvent& outEvent) noexcept { float sumSq = 0.0f; float peakHopSample = 0.0f; for (int i = 0; i < 256; ++i) { sumSq += hop256[i] * hop256[i]; peakHopSample = std::max(peakHopSample, std::fabs(hop256[i])); } if (peakHopSample >= 0.985f) clipDetected_ = true; const float rmsDb = 10.0f * std::log10(sumSq / 256.0f + 1e-12f); const float deltaDb = rmsDb - prevHopDb_; prevHopDb_ = rmsDb; if (cooldownHops_ > 0) { --cooldownHops_; return false; } if (state_ == State::MeasureNoise) { noiseEnergySum_ += rmsDb; if (++noiseHops_ >= 30) { profile_.ambientNoiseDb = noiseEnergySum_ / 30.0f; profile_.calibratedGateDb = std::clamp(profile_.ambientNoiseDb + 6.0f, -58.0f, -36.0f); state_ = State::WaitStrike; emitEvent(outEvent, 1, 0.0f, 0.0f, profile_.ambientNoiseDb, 0.0f); return true; } return false; } if (state_ == State::Done) return false; const int targetMidi = fullMode_ ? kFullChromaticSeq[keyIdx_] : kFastSplitScaleSeq[keyIdx_]; const float nomFreq = 440.0f * std::pow(2.0f, (targetMidi - 69) / 12.0f); if (state_ == State::WaitReleaseValley) { valleyFloorDb_ = std::min(valleyFloorDb_, rmsDb); const bool droppedEnough = (valleyFloorDb_ < peakRmsDb_ - 5.0f) ¦¦ (rmsDb < profile_.calibratedGateDb + 2.0f); const bool freshAttack = (deltaDb > 3.2f && rmsDb > valleyFloorDb_ + 4.2f); if (droppedEnough && freshAttack) { state_ = State::SettleStrike; settleCount_ = 0; peakRmsDb_ = rmsDb; } return false; } if (state_ == State::WaitStrike) { if (rmsDb > profile_.calibratedGateDb + 3.0f && deltaDb > 2.8f) { state_ = State::SettleStrike; settleCount_ = 0; peakRmsDb_ = rmsDb; } return false; } if (state_ == State::SettleStrike) { peakRmsDb_ = std::max(peakRmsDb_, rmsDb); if (++settleCount_ >= 3) { for (int i = 0; i < 4096; ++i) { win4096_[i] = window4096[i] * hann4096_[i]; } const int verifyStatus = analyzeStrike(win4096_, targetMidi, nomFreq, strikeIdx_, outEvent); if (verifyStatus < 0) { outEvent.state = verifyStatus; cooldownHops_ = 24; state_ = (strikeIdx_ == 1) ? State::WaitStrike : State::WaitReleaseValley; valleyFloorDb_ = peakRmsDb_; return true; } cooldownHops_ = 16; if (strikeIdx_ == 1) { strikeIdx_ = 2; valleyFloorDb_ = peakRmsDb_; state_ = State::WaitReleaseValley; outEvent.state = 2; // Prompt for Strike 2 (Firm) } else { finalizeKey(targetMidi); if (++keyIdx_ >= totalKeys_) { finalizeProfile(); state_ = State::Done; outEvent.state = 4; // Complete } else { strikeIdx_ = 1; state_ = State::WaitStrike; outEvent.state = 3; // Next Key } } return true; } } return false; } const PianoProfile& profile() const noexcept { return profile_; } static std::vector<float> serializeProfile(const PianoProfile& p) { std::vector<float> out; out.reserve(539); out.push_back(p.isCalibrated ? 1.0f : 0.0f); out.push_back(static_cast<float>(p.minCalibratedMidi)); out.push_back(static_cast<float>(p.maxCalibratedMidi)); out.push_back(p.a4ReferenceHz); out.push_back(p.ambientNoiseDb); out.push_back(p.calibratedGateDb); out.push_back(p.inputGainDb); out.push_back(p.inputGainLinear); out.push_back(p.calibratedPitchWindowCents); out.push_back(p.minCombEnergyRatio); out.push_back(p.minTimbreSimilarity); for (int m = 0; m < 128; ++m) out.push_back(p.keyFreqHz[m]); for (int m = 0; m < 128; ++m) out.push_back(p.inharmonicityB[m]); for (int m = 0; m < 128; ++m) out.push_back(p.keySoftDb[m]); for (int m = 0; m < 128; ++m) out.push_back(p.keyLoudDb[m]); for (int i = 0; i < 8; ++i) out.push_back(p.softPartialWeights[i]); for (int i = 0; i < 8; ++i) out.push_back(p.loudPartialWeights[i]); return out; } static bool deserializeProfile(const std::vector<float>& data, PianoProfile& out) noexcept { if (data.size() < 539) return false; size_t idx = 0; out.isCalibrated = (data[idx++] > 0.5f); out.minCalibratedMidi = static_cast<int>(data[idx++]); out.maxCalibratedMidi = static_cast<int>(data[idx++]); out.a4ReferenceHz = data[idx++]; out.ambientNoiseDb = data[idx++]; out.calibratedGateDb = data[idx++]; out.inputGainDb = data[idx++]; out.inputGainLinear = data[idx++]; out.calibratedPitchWindowCents = data[idx++]; out.minCombEnergyRatio = data[idx++]; out.minTimbreSimilarity = data[idx++]; for (int m = 0; m < 128; ++m) out.keyFreqHz[m] = data[idx++]; for (int m = 0; m < 128; ++m) out.inharmonicityB[m] = data[idx++]; for (int m = 0; m < 128; ++m) out.keySoftDb[m] = data[idx++]; for (int m = 0; m < 128; ++m) out.keyLoudDb[m] = data[idx++]; for (int i = 0; i < 8; ++i) out.softPartialWeights[i] = data[idx++]; for (int i = 0; i < 8; ++i) out.loudPartialWeights[i] = data[idx++]; return true; } private: enum class State { MeasureNoise, WaitStrike, WaitReleaseValley, SettleStrike, Done }; float findPeakFrequency(const float* win4096, float centerHz, float rangeCents, float& outMeanSqPower) const noexcept { constexpr int kSteps = 41; float bestFreq = centerHz; float bestMagSq = -1.0f; for (int i = 0; i < kSteps; ++i) { const float c = -rangeCents + (2.0f * rangeCents * i) / (kSteps - 1); const float candHz = centerHz * std::pow(2.0f, c / 1200.0f); const float magSq = Goertzel::magnitudeSquared(win4096, 4096, candHz, sr_); if (magSq > bestMagSq) { bestMagSq = magSq; bestFreq = candHz; } } outMeanSqPower = Goertzel::toMeanSquarePower(std::max(0.0f, bestMagSq), 4096); return bestFreq; } int analyzeStrike(const float* win4096, int midi, float nomFreq, int strikeNum, CalibrationProgressEvent& ev) noexcept { float p1 = 0.0f, p2 = 0.0f, p3 = 0.0f; float f1 = findPeakFrequency(win4096, nomFreq, 60.0f, p1); const float f2Est = findPeakFrequency(win4096, 2.0f * nomFreq * 1.0007f, 55.0f, p2); const float f3Est = findPeakFrequency(win4096, 3.0f * nomFreq * 1.0016f, 55.0f, p3); // Missing-Fundamental Recovery for Deep Bass (e.g., C1 at 32.7 Hz on small uprights/phone mics) if (midi <= 36 && p1 < 0.25f * p2 && p2 > 1e-9f) { f1 = f2Est / (2.0f * std::sqrt(1.0f + 4.0f * 0.00045f)); } const float targetCompositePower = (midi <= 36) ? std::max(p1, 0.7f * (p2 + p3)) : p1; // Wrong-key detection using composite harmonic power for (int offset : {-2, -1, 1, 2}) { float otherP1 = 0.0f, otherP2 = 0.0f; const float otherNom = 440.0f * std::pow(2.0f, (midi + offset - 69) / 12.0f); findPeakFrequency(win4096, otherNom, 35.0f, otherP1); if (midi <= 36) { findPeakFrequency(win4096, 2.0f * otherNom, 35.0f, otherP2); } const float otherComposite = (midi <= 36) ? std::max(otherP1, 0.7f * otherP2) : otherP1; if (otherComposite > targetCompositePower * 1.85f) { emitEvent(ev, strikeNum, otherNom, 0.0f, peakRmsDb_, 0.0f); return -1; } } float bSum = 0.0f; int bCount = 0; float overtonePowerSum = 0.0f; const float refAmp1 = std::sqrt(std::max(p1, 0.35f * p2) + 1e-12f); for (int h = 2; h <= 6; ++h) { float pH = 0.0f; const float expectedH = h * f1 * std::sqrt(1.0f + 0.00035f * static_cast<float>(h * h)); const float fh = findPeakFrequency(win4096, expectedH, 45.0f, pH); if (h <= 4) overtonePowerSum += pH; const float ratioSq = (fh / (h * f1)) * (fh / (h * f1)); if (ratioSq > 0.999f && ratioSq < 1.09f) { bSum += (ratioSq - 1.0f) / static_cast<float>(h * h); ++bCount; } const float ampH = std::sqrt(pH); if (refAmp1 > 1e-6f) { const float w = std::clamp(ampH / refAmp1, 0.00f, 2.5f); if (strikeNum == 1) { profile_.softPartialWeights[h - 1] = 0.7f * profile_.softPartialWeights[h - 1] + 0.3f * w; } else { profile_.loudPartialWeights[h - 1] = 0.7f * profile_.loudPartialWeights[h - 1] + 0.3f * w; } } } const float overtoneRatio = overtonePowerSum / (targetCompositePower + 1e-12f); if (strikeNum == 2 && overtoneRatio < 0.030f) { emitEvent(ev, strikeNum, f1, 0.0f, peakRmsDb_, 0.0f); return -2; // Reject pure sine whistle } const float bMeasured = (bCount > 0) ? (bSum / bCount) : 0.00035f; const float cents = 1200.0f * std::log2(f1 / nomFreq); const float partialDb = 10.0f * std::log10(targetCompositePower + 1e-12f); if (strikeNum == 1) { f1Soft_ = f1; bSoft_ = bMeasured; rmsSoft_ = partialDb; } else { f1Loud_ = f1; bLoud_ = bMeasured; rmsLoud_ = std::max(partialDb, rmsSoft_ + 6.0f); rawFirmPeakMax_ = std::max(rawFirmPeakMax_, peakRmsDb_); } emitEvent(ev, strikeNum, f1, cents, partialDb, bMeasured); return 0; } void finalizeKey(int midi) noexcept { profile_.keyFreqHz[midi] = 0.5f * (f1Soft_ + f1Loud_); profile_.inharmonicityB[midi] = 0.3f * bSoft_ + 0.7f * bLoud_; profile_.keySoftDb[midi] = rmsSoft_; profile_.keyLoudDb[midi] = rmsLoud_; measuredKeyMask_[midi] = true; } void finalizeProfile() noexcept { // 1. Auto-compute digital gain staging to normalize firm strikes to -10 dBFS profile_.inputGainDb = std::clamp(-10.0f - rawFirmPeakMax_, -12.0f, +24.0f); profile_.inputGainLinear = std::pow(10.0f, profile_.inputGainDb / 20.0f); // 2. Shift all measured dB thresholds by +inputGainDb to match post-gain signal in Cascade.h profile_.ambientNoiseDb += profile_.inputGainDb; profile_.calibratedGateDb = std::clamp(profile_.ambientNoiseDb + 6.0f, -56.0f, -32.0f); for (int m = 21; m <= 108; ++m) { if (measuredKeyMask_[m]) { profile_.keySoftDb[m] += profile_.inputGainDb; profile_.keyLoudDb[m] += profile_.inputGainDb; } } if (measuredKeyMask_[69]) { profile_.a4ReferenceHz = profile_.keyFreqHz[69]; } else if (measuredKeyMask_[60]) { const float et60 = 440.0f * std::pow(2.0f, (60 - 69) / 12.0f); profile_.a4ReferenceHz = 440.0f * (profile_.keyFreqHz[60] / et60); } // 3. Split-scale interpolation across the C3 (MIDI 48) bridge break const float b24 = std::max(1e-5f, profile_.inharmonicityB[24]); const float b36 = std::max(1e-5f, profile_.inharmonicityB[36]); const float bassSlope = std::clamp(std::log(b24 / b36) / 12.0f, 0.01f, 0.06f); const float b60 = std::max(1e-5f, profile_.inharmonicityB[60]); const float bTop = measuredKeyMask_[84] ? std::max(1e-5f, profile_.inharmonicityB[84]) : std::max(1e-5f, profile_.inharmonicityB[72]); const float spanSemis = measuredKeyMask_[84] ? 24.0f : 12.0f; const float trebleSlope = std::clamp(std::log(bTop / b60) / spanSemis, 0.02f, 0.065f); for (int m = 21; m <= 108; ++m) { if (!measuredKeyMask_[m]) { if (m < 48) { profile_.inharmonicityB[m] = std::clamp(b36 * std::exp(bassSlope * (36.0f - m)), 0.00005f, 0.015f); profile_.keySoftDb[m] = profile_.keySoftDb[36] - 0.20f * (36 - m); profile_.keyLoudDb[m] = profile_.keyLoudDb[36] - 0.20f * (36 - m); } else { profile_.inharmonicityB[m] = std::clamp(b60 * std::exp(trebleSlope * (m - 60.0f)), 0.00005f, 0.025f); profile_.keySoftDb[m] = profile_.keySoftDb[60] - 0.18f * (m - 60); profile_.keyLoudDb[m] = profile_.keyLoudDb[60] - 0.18f * (m - 60); } const float hTune = (m < 48) ? 3.0f : 2.0f; const float stretchFactor = std::sqrt((1.0f + profile_.inharmonicityB[m] * hTune * hTune) / (1.0f + b60 * hTune * hTune)); const float direction = (m < 60) ? (1.0f / stretchFactor) : stretchFactor; profile_.keyFreqHz[m] = profile_.a4ReferenceHz * std::pow(2.0f, (m - 69) / 12.0f) * direction; } } profile_.isCalibrated = true; } void emitEvent(CalibrationProgressEvent& ev, int strikeNum, float freq, float cents, float rmsDb, float bVal) const noexcept { ev.currentMidi = fullMode_ ? kFullChromaticSeq[keyIdx_] : kFastSplitScaleSeq[keyIdx_]; ev.keyIndex = keyIdx_; ev.totalKeys = totalKeys_; ev.strikeNumber = strikeNum; ev.state = (state_ == State::MeasureNoise) ? 1 : strikeNum; ev.measuredFreqHz = freq; ev.centsOffset = cents; ev.strikeRmsDb = rmsDb; ev.measuredB = bVal; ev.recommendedGainDb = profile_.inputGainDb; ev.isClipping = clipDetected_; } int sr_ = 48000; bool fullMode_ = false; int totalKeys_ = 7; int keyIdx_ = 0; int strikeIdx_ = 1; State state_ = State::MeasureNoise; int noiseHops_ = 0; float noiseEnergySum_ = 0.0f; float prevHopDb_ = -60.0f; float valleyFloorDb_ = -60.0f; int settleCount_ = 0; int cooldownHops_ = 0; float peakRmsDb_ = -60.0f; float rawFirmPeakMax_ = -60.0f; bool clipDetected_ = false; float f1Soft_ = 0.0f, f1Loud_ = 0.0f; float bSoft_ = 0.0f, bLoud_ = 0.0f; float rmsSoft_ = -44.0f, rmsLoud_ = -10.0f; std::array<bool, 128> measuredKeyMask_{}; PianoProfile profile_{}; alignas(64) float hann4096_[4096]{}; alignas(64) float win4096_[4096]{}; }; } // namespace piano

v2.0 Apex Step 4: Hardware Audio Interface & Real-Time Worker Engine (shared/AudioInput.h & shared/Engine.h)

shared/AudioInput.h & shared/Engine.h (v2.0 Apex Production Architecture)

// shared/AudioInput.h (v2.0 Apex Production Architecture) #pragma once #include <cstddef> #include <memory> namespace piano { class AudioInputCallback { public: virtual ~AudioInputCallback() = default; virtual void onAudioSamples(const float* samples, std::size_t count) noexcept = 0; }; class AudioInput { public: virtual ~AudioInput() = default; virtual bool start(int sampleRate, int framesPerBurst, AudioInputCallback* cb) = 0; virtual void stop() = 0; }; std::unique_ptr<AudioInput> createPlatformAudioInput(); } // namespace piano // ============================================================================ // shared/Engine.h (v2.0 Apex Production Architecture) // ============================================================================ #pragma once #include "AudioInput.h" #include "Calibration.h" #include "Cascade.h" #include "SampleRing.h" #include <atomic> #include <chrono> #include <condition_variable> #include <functional> #include <mutex> #include <thread> #include <vector> namespace piano { struct EngineStats { double avgUs = 0.0; double maxUs = 0.0; uint32_t lateHops = 0; uint32_t clipCount = 0; float tuningDriftCents = 0.0f; bool isCalibrated = false; }; class Engine final : public AudioInputCallback { public: using NoteCallback = std::function<void(const NoteEvent&)>; using CalibrationCallback = std::function<void(const CalibrationProgressEvent&)>; Engine() : audioInput_(createPlatformAudioInput()) { outEvents_.reserve(16); } ~Engine() override { stop(); } bool startDetector(NoteCallback onNote) { stop(); noteCb_ = std::move(onNote); calibrating_.store(false, std::memory_order_release); resetTelemetry(); running_.store(true, std::memory_order_release); workerThread_ = std::thread(&Engine::workerLoop, this); return audioInput_->start(48000, 256, this); } bool startCalibration(bool fullChromaticMode, CalibrationCallback onProgress) { calibrator_.start(48000, fullChromaticMode); calCb_ = std::move(onProgress); calibrating_.store(true, std::memory_order_release); if (!running_.load(std::memory_order_acquire)) { resetTelemetry(); running_.store(true, std::memory_order_release); workerThread_ = std::thread(&Engine::workerLoop, this); return audioInput_->start(48000, 256, this); } return true; } void stop() { if (!running_.exchange(false, std::memory_order_acq_rel)) return; cv_.notify_all(); if (audioInput_) audioInput_->stop(); if (workerThread_.joinable()) workerThread_.join(); } void setExpectedNotes(const std::vector<int>& midiNotes) noexcept { cascade_.setExpectedNotes(midiNotes); } std::vector<float> exportProfile() const { return PianoCalibrator::serializeProfile(cascade_.profile()); } bool importProfile(const std::vector<float>& serialized) noexcept { PianoProfile prof{}; if (!PianoCalibrator::deserializeProfile(serialized, prof)) return false; cascade_.setProfile(prof); return true; } EngineStats getStats() const noexcept { const uint64_t hops = totalHops_.load(std::memory_order_relaxed); const uint64_t sumNs = sumExecNs_.load(std::memory_order_relaxed); const uint64_t maxNs = maxExecNs_.load(std::memory_order_relaxed); EngineStats st{}; st.avgUs = (hops > 0) ? (static_cast<double>(sumNs) / static_cast<double>(hops)) * 1e-3 : 0.0; st.maxUs = static_cast<double>(maxNs) * 1e-3; st.lateHops = lateHops_.load(std::memory_order_relaxed); st.clipCount = cascade_.clipCount(); st.tuningDriftCents = cascade_.estimatedTuningDriftCents(); st.isCalibrated = cascade_.profile().isCalibrated; return st; } void onAudioSamples(const float* samples, std::size_t count) noexcept override { ring_.push(samples, count); if (ring_.available() >= 256) { cv_.notify_one(); } } private: void resetTelemetry() noexcept { totalHops_.store(0, std::memory_order_relaxed); sumExecNs_.store(0, std::memory_order_relaxed); maxExecNs_.store(0, std::memory_order_relaxed); lateHops_.store(0, std::memory_order_relaxed); hopCounter_ = 0; } void workerLoop() { constexpr std::size_t kHopSize = 256; constexpr std::size_t kWinSize = 4096; while (running_.load(std::memory_order_acquire)) { if (ring_.available() < kHopSize) { std::unique_lock<std::mutex> lk(wakeMutex_); cv_.wait_for(lk, std::chrono::milliseconds(5), [this] { return !running_.load(std::memory_order_relaxed) ¦¦ ring_.available() >= kHopSize; }); continue; } if (ring_.dropStaleBacklog(kHopSize, 4)) { lateHops_.fetch_add(1, std::memory_order_relaxed); } const auto t0 = std::chrono::steady_clock::now(); ring_.peekWindow(window4096_, kWinSize, kHopSize); ring_.consume(kHopSize); const float* hop256 = window4096_ + (kWinSize - kHopSize); const double streamTimeMs = static_cast<double>(hopCounter_++) * (256000.0 / 48000.0); if (calibrating_.load(std::memory_order_acquire)) { CalibrationProgressEvent calEv{}; if (calibrator_.processWindow(window4096_, hop256, calEv)) { if (calEv.state == 4) { cascade_.setProfile(calibrator_.profile()); calibrating_.store(false, std::memory_order_release); } if (calCb_) calCb_(calEv); } } else { outEvents_.clear(); cascade_.processHop(window4096_, hop256, streamTimeMs, outEvents_); if (noteCb_) { for (const auto& ev : outEvents_) { noteCb_(ev); } } } const auto t1 = std::chrono::steady_clock::now(); const uint64_t elapsedNs = static_cast<uint64_t>(std::chrono::duration_cast<std::chrono::nanoseconds>(t1 - t0).count()); totalHops_.fetch_add(1, std::memory_order_relaxed); sumExecNs_.fetch_add(elapsedNs, std::memory_order_relaxed); uint64_t prevMax = maxExecNs_.load(std::memory_order_relaxed); while (elapsedNs > prevMax && !maxExecNs_.compare_exchange_weak(prevMax, elapsedNs, std::memory_order_relaxed)) {} if (elapsedNs > 5333000ULL) { lateHops_.fetch_add(1, std::memory_order_relaxed); } } } std::unique_ptr<AudioInput> audioInput_; SampleRing<16384> ring_; Cascade cascade_; PianoCalibrator calibrator_; alignas(64) float window4096_[4096]{}; std::vector<NoteEvent> outEvents_; std::atomic<bool> running_{false}; std::atomic<bool> calibrating_{false}; std::thread workerThread_; std::mutex wakeMutex_; std::condition_variable cv_; NoteCallback noteCb_; CalibrationCallback calCb_; uint64_t hopCounter_ = 0; std::atomic<uint64_t> totalHops_{0}; std::atomic<uint64_t> sumExecNs_{0}; std::atomic<uint64_t> maxExecNs_{0}; std::atomic<uint32_t> lateHops_{0}; }; } // namespace piano

v2.0 Apex Step 5: React Native Codegen TurboModule Specification & C++ JSI Bindings

specs/NativePianoEngine.ts & shared/NativePianoEngine.{h,cpp} (v2.0 Apex)

// specs/NativePianoEngine.ts (v2.0 Apex Production Architecture) import type {TurboModule} from 'react-native'; import {TurboModuleRegistry} from 'react-native'; export type NoteEventPayload = { type: number; // 1 = NoteOn, 0 = NoteOff midi: number; // 21..108 cents: number; // Fine tuning offset in cents velocity: number; // 0.05..1.00 hybrid PHE + brightness velocity tMs: number; // Stream timestamp in milliseconds }; export type CalibrationEventPayload = { currentMidi: number; keyIndex: number; totalKeys: number; strikeNumber: number; state: number; measuredFreqHz: number; centsOffset: number; strikeRmsDb: number; measuredB: number; recommendedGainDb: number; isClipping: boolean; }; export type EngineStatsPayload = { avgUs: number; maxUs: number; lateHops: number; clipCount: number; tuningDriftCents: number; isCalibrated: boolean; }; export interface Spec extends TurboModule { startDetector(onNote: (event: NoteEventPayload) => void): boolean; stopDetector(): void; setExpectedNotes(midiNotes: ReadonlyArray<number>): void; startCalibration( fullChromaticMode: boolean, onProgress: (event: CalibrationEventPayload) => void, ): boolean; exportProfile(): ReadonlyArray<number>; importProfile(serializedProfile: ReadonlyArray<number>): boolean; getEngineStats(): EngineStatsPayload; } export default TurboModuleRegistry.getEnforcing<Spec>('NativePianoEngine'); // ============================================================================ // shared/NativePianoEngine.h & shared/NativePianoEngine.cpp (v2.0 Apex) // ============================================================================ #pragma once #include <AppSpecsJSI.h> #include "Engine.h" #include <memory> namespace facebook::react { class NativePianoEngine : public NativePianoEngineCxxSpec<NativePianoEngine> { public: explicit NativePianoEngine(std::shared_ptr<CallInvoker> jsInvoker) : NativePianoEngineCxxSpec(std::move(jsInvoker)) {} ~NativePianoEngine() override { engine_.stop(); } bool startDetector(jsi::Runtime& /*rt*/, AsyncCallback<jsi::Object> onNote) { return engine_.startDetector([cb = std::move(onNote)](const piano::NoteEvent& ev) mutable { cb.call([ev](jsi::Runtime& rt, jsi::Function& jsFn) { jsi::Object obj(rt); obj.setProperty(rt, "type", static_cast<int>(ev.type)); obj.setProperty(rt, "midi", ev.midi); obj.setProperty(rt, "cents", static_cast<double>(ev.cents)); obj.setProperty(rt, "velocity", static_cast<double>(ev.velocity)); obj.setProperty(rt, "tMs", ev.tMs); jsFn.call(rt, obj); }); }); } void stopDetector(jsi::Runtime& /*rt*/) { engine_.stop(); } void setExpectedNotes(jsi::Runtime& /*rt*/, std::vector<int> midiNotes) { engine_.setExpectedNotes(midiNotes); } bool startCalibration(jsi::Runtime& /*rt*/, bool fullChromaticMode, AsyncCallback<jsi::Object> onProgress) { return engine_.startCalibration( fullChromaticMode, [cb = std::move(onProgress)](const piano::CalibrationProgressEvent& ev) mutable { cb.call([ev](jsi::Runtime& rt, jsi::Function& jsFn) { jsi::Object obj(rt); obj.setProperty(rt, "currentMidi", ev.currentMidi); obj.setProperty(rt, "keyIndex", ev.keyIndex); obj.setProperty(rt, "totalKeys", ev.totalKeys); obj.setProperty(rt, "strikeNumber", ev.strikeNumber); obj.setProperty(rt, "state", ev.state); obj.setProperty(rt, "measuredFreqHz", static_cast<double>(ev.measuredFreqHz)); obj.setProperty(rt, "centsOffset", static_cast<double>(ev.centsOffset)); obj.setProperty(rt, "strikeRmsDb", static_cast<double>(ev.strikeRmsDb)); obj.setProperty(rt, "measuredB", static_cast<double>(ev.measuredB)); obj.setProperty(rt, "recommendedGainDb", static_cast<double>(ev.recommendedGainDb)); obj.setProperty(rt, "isClipping", ev.isClipping); jsFn.call(rt, obj); }); }); } std::vector<double> exportProfile(jsi::Runtime& /*rt*/) { const auto floats = engine_.exportProfile(); return std::vector<double>(floats.begin(), floats.end()); } bool importProfile(jsi::Runtime& /*rt*/, std::vector<double> serializedProfile) { std::vector<float> floats(serializedProfile.begin(), serializedProfile.end()); return engine_.importProfile(floats); } jsi::Object getEngineStats(jsi::Runtime& rt) { const auto st = engine_.getStats(); jsi::Object obj(rt); obj.setProperty(rt, "avgUs", st.avgUs); obj.setProperty(rt, "maxUs", st.maxUs); obj.setProperty(rt, "lateHops", static_cast<double>(st.lateHops)); obj.setProperty(rt, "clipCount", static_cast<double>(st.clipCount)); obj.setProperty(rt, "tuningDriftCents", static_cast<double>(st.tuningDriftCents)); obj.setProperty(rt, "isCalibrated", st.isCalibrated); return obj; } private: piano::Engine engine_; }; } // namespace facebook::react

v2.0 Apex Step 6: Guided Dynamic React Native UI (src/CalibrationScreen.tsx)

src/CalibrationScreen.tsx (v2.0 Apex Production Architecture)

// src/CalibrationScreen.tsx (v2.0 Apex Production Architecture) import React, {useState, useEffect} from 'react'; import {View, Text, TouchableOpacity, StyleSheet} from 'react-native'; export type CalibrationStepEvent = { currentMidi: number; keyIndex: number; totalKeys: number; strikeNumber: number; // 1 (Soft 5%-20%) or 2 (Firm 75%-100%) state: number; // -2=Noise, -1=WrongKey, 0=Noise, 1=Wait1, 2=Wait2, 3=KeyDone, 4=AllDone measuredFreqHz: number; centsOffset: number; strikeRmsDb: number; measuredB: number; recommendedGainDb: number; isClipping: boolean; }; const NOTE_NAMES: Record<number, {label: string; hint: string}> = { 24: {label: 'C1 (Deep Bass C)', hint: 'Lowest C on the left side (copper-wound bass bridge)'}, 36: {label: 'C2 (Bass C)', hint: 'Two octaves below Middle C (octave 2 baseline)'}, 48: {label: 'C3 (Bridge Break C)', hint: 'One octave below Middle C (bass-to-treble break)'}, 60: {label: 'C4 (Middle C)', hint: 'Center white key just left of two black keys'}, 61: {label: 'C#4', hint: 'First black key to the right of Middle C'}, 62: {label: 'D4', hint: 'White key between C#4 and D#4'}, 63: {label: 'D#4', hint: 'Second black key in the two-black-key group'}, 64: {label: 'E4', hint: 'White key immediately right of D#4'}, 65: {label: 'F4', hint: 'White key just left of three black keys'}, 66: {label: 'F#4', hint: 'First black key in three-black-key group'}, 67: {label: 'G4', hint: 'White key between F#4 and G#4'}, 68: {label: 'G#4', hint: 'Middle black key in three-black-key group'}, 69: {label: 'A4 (440 Hz Reference)', hint: 'Five white keys above Middle C (tuning reference)'}, 70: {label: 'A#4', hint: 'Third black key in three-black-key group'}, 71: {label: 'B4', hint: 'White key immediately right of A#4'}, 72: {label: 'C5 (Treble C)', hint: 'One octave above Middle C (treble stretch)'}, 84: {label: 'C6 (High Treble C)', hint: 'Two octaves above Middle C (high treble stiffness)'}, }; export function CalibrationScreen({ onStartCalibration, onComplete, latestEvent, }: { onStartCalibration: (fullChromaticMode: boolean) => void; onComplete: () => void; latestEvent?: CalibrationStepEvent; }) { const [started, setStarted] = useState(false); const [fullMode, setFullMode] = useState(false); const midi = latestEvent?.currentMidi ?? 24; const noteMeta = NOTE_NAMES[midi] ?? {label: `MIDI ${midi}`, hint: ''}; const strikeNum = latestEvent?.state === 2 ? 2 : 1; const isAllDone = latestEvent?.state === 4; useEffect(() => { if (isAllDone) onComplete(); }, [isAllDone, onComplete]); return ( <View style={styles.container}> <Text style={styles.title}>Piano Profile & Intensity Calibration (v2.0 Apex)</Text> <Text style={styles.subtitle}> Strike each key <Text style={styles.bold}>2 times</Text>: Strike 1 sets your 5% soft-touch sensitivity and fundamental pitch, and Strike 2 sets your 100% hard-strike dynamic ceiling and string stiffness. </Text> {!started ? ( <View style={styles.modeContainer}> <TouchableOpacity style={styles.primaryBtn} onPress={() => { setFullMode(false); setStarted(true); onStartCalibration(false); }}> <Text style={styles.primaryBtnText}> Fast 88-Key Split-Scale (Recommended • 7 Keys: C1–C6 + A4) </Text> </TouchableOpacity> <TouchableOpacity style={styles.secondaryBtn} onPress={() => { setFullMode(true); setStarted(true); onStartCalibration(true); }}> <Text style={styles.secondaryBtnText}> Full Chromatic Middle Octave + Multi-Octave Anchors (16 Keys) </Text> </TouchableOpacity> </View> ) : ( <View style={styles.card}> <Text style={styles.stepCounter}> Key {(latestEvent?.keyIndex ?? 0) + 1} of {latestEvent?.totalKeys ?? (fullMode ? 16 : 7)} </Text> <Text style={styles.targetKey}>{noteMeta.label}</Text> <Text style={styles.keyHint}>{noteMeta.hint}</Text> {latestEvent?.isClipping && ( <View style={styles.warningBanner}> <Text style={styles.warningText}> Microphone clipping detected! Lower keyboard volume or move device slightly back. </Text> </View> )} {latestEvent?.state === -1 && ( <View style={styles.warningBanner}> <Text style={styles.warningText}> Wrong key detected! Please strike {noteMeta.label} only. </Text> </View> )} {latestEvent?.state === -2 && ( <View style={styles.warningBanner}> <Text style={styles.warningText}> Non-piano sound or whistle ignored. Please play the acoustic key. </Text> </View> )} <View style={styles.strikeRow}> <View style={[styles.strikeBadge, strikeNum === 1 && styles.strikeBadgeActive]}> <Text style={styles.strikeTitle}>Strike 1 of 2: Gentle Soft Touch (5%–20%)</Text> <Text style={styles.strikeDesc}> Press {noteMeta.label.split(' ')[0]} softly to calibrate soft-touch sensitivity and dark harmonic timbre. </Text> </View> <View style={[styles.strikeBadge, strikeNum === 2 && styles.strikeBadgeActive]}> <Text style={styles.strikeTitle}>Strike 2 of 2: Firm Strike (75%–100%)</Text> <Text style={styles.strikeDesc}> Drop your finger firmly on {noteMeta.label.split(' ')[0]} to measure string stiffness, overtone brightness, and mic gain headroom. </Text> </View> </View> {latestEvent && latestEvent.measuredFreqHz > 0 && latestEvent.state > 0 && ( <View style={styles.telemetryBox}> <Text style={styles.telemetryText}> Pitch: {latestEvent.measuredFreqHz.toFixed(2)} Hz ( {latestEvent.centsOffset >= 0 ? '+' : ''} {latestEvent.centsOffset.toFixed(1)} cents) ¦ Partial Power: {latestEvent.strikeRmsDb.toFixed(1)} dBFS </Text> <Text style={styles.telemetryText}> String Stiffness B: {latestEvent.measuredB.toFixed(5)} ¦ Mic Pre-Gain:{' '} {latestEvent.recommendedGainDb >= 0 ? '+' : ''} {latestEvent.recommendedGainDb.toFixed(1)} dB </Text> <Text style={styles.profileBadge}> {latestEvent.strikeRmsDb > -12.0 ? 'High Dynamic Input (Fortissimo Headroom OK)' : 'Normal/Soft Input (5% Detection Enabled)'} </Text> </View> )} </View> )} </View> ); } const styles = StyleSheet.create({ container: {flex: 1, padding: 24, backgroundColor: '#121316', justifyContent: 'center'}, title: {fontSize: 24, fontWeight: '700', color: '#FFFFFF', marginBottom: 8}, subtitle: {fontSize: 14, color: '#A0A6B2', lineHeight: 20, marginBottom: 24}, bold: {color: '#FFFFFF', fontWeight: '700'}, modeContainer: {flexDirection: 'column', gap: 12}, primaryBtn: {backgroundColor: '#1A73E8', paddingVertical: 16, paddingHorizontal: 14, borderRadius: 10, alignItems: 'center'}, primaryBtnText: {color: '#FFFFFF', fontSize: 15, fontWeight: '600', textAlign: 'center'}, secondaryBtn: {backgroundColor: '#1E2025', paddingVertical: 14, paddingHorizontal: 14, borderRadius: 10, alignItems: 'center', borderWidth: 1, borderColor: '#3C4043'}, secondaryBtnText: {color: '#8AB4F8', fontSize: 14, fontWeight: '600', textAlign: 'center'}, card: {backgroundColor: '#1E2025', borderRadius: 14, padding: 20, borderWidth: 1, borderColor: '#2E323B'}, stepCounter: {fontSize: 12, fontWeight: '600', color: '#8AB4F8', textTransform: 'uppercase'}, targetKey: {fontSize: 30, fontWeight: '700', color: '#FFFFFF', marginTop: 6}, keyHint: {fontSize: 13, color: '#A0A6B2', marginTop: 4, marginBottom: 16}, warningBanner: {backgroundColor: '#371C1B', borderColor: '#F28B82', borderWidth: 1, borderRadius: 8, padding: 10, marginBottom: 14}, warningText: {color: '#F28B82', fontSize: 13, fontWeight: '600'}, strikeRow: {flexDirection: 'column', gap: 12}, strikeBadge: {padding: 14, borderRadius: 10, backgroundColor: '#141619', borderWidth: 1, borderColor: '#2B2F36'}, strikeBadgeActive: {borderColor: '#1A73E8', backgroundColor: '#17223B'}, strikeTitle: {fontSize: 15, fontWeight: '700', color: '#FFFFFF', marginBottom: 4}, strikeDesc: {fontSize: 13, color: '#C4C9D4', lineHeight: 18}, telemetryBox: {marginTop: 18, paddingTop: 14, borderTopWidth: 1, borderTopColor: '#2E323B'}, telemetryText: {fontSize: 12, color: '#A0A6B2', fontFamily: 'monospace', marginBottom: 4}, profileBadge: {fontSize: 13, fontWeight: '600', color: '#81C995', marginTop: 4}, });

8. Acoustic Calibration and Deployment Recommendations

While the portable C++ core achieves 100% single-note accuracy (88/88 keys) and sub-35 µs mean hop execution on synthetic grand-piano benchmarks, deploying to physical mobile hardware in reverberant rooms requires targeted operational verification of shared/Cascade.h [1].

Real-Device Telemetry Verification

Because baseline desktop benchmarks execute on x86/ARM desktop harnesses rather than physical phones, engineers should immediately poll getEngineStats() (avgUs, maxUs, lateHops, clipCount, and tuningDriftCents) on target Android and iOS handsets during live acoustic performance [1]. Monitoring these telemetry values verifies that worst-case execution (maxUs) remains well below the 5333 µs hop deadline and that lateHops remains zero even under mobile thermal throttling or background OS activity [1]. Engineers must also account for the fact that event timestamps (tMs) measure stream time elapsed since startDetector() and do not include OS or hardware microphone input buffer latency, which must be calibrated as a constant offset when scoring rhythmic timing accuracy [1].

Resolving Octave Ambiguity in Expected-Notes and Free-Form Modes

A known physical ambiguity occurs in v1.0 setExpectedNotes mode whenever the expected note set contains both a pitch f₀ and its octave above 2f₀ (for example, C3 and C4) [1]. Playing the lower note (C3) alone in v1.0 could fire a false-positive noteOn for the upper note (C4) because C3's second partial (2f₀) coincides directly with C4's fundamental frequency, and C3's fourth partial (4f₀) coincides with C4's second partial [1]. In v2.0 Apex, Cascade.h resolves this ambiguity both across hops (via physical partial power subtraction voices_[v].partialPower[vh]) and within the same hop (via the odd-harmonic disambiguation bonus (P₃ + P₅)/(P₂ + P₄) and intra-hop Klapuri peeling), ensuring the lower pitch f₀ is extracted first and its exact overtone power is subtracted before 2f₀ is evaluated [28].

Calibrating for Small Upright Pianos and Hybrid Neural Filtering

Short upright pianos exhibit up to 4× higher string stiffness (B) than concert grands, stretching upper partials enough to reduce uncalibrated v1.0 single-note bass detection from 27/27 to 24/27 and inducing ghost notes in the experimental chord stage [1]. Using the v2.0 Apex Split-Scale Calibration curve in Calibration.h—with 2f₀/3f₀ missing-fundamental recovery on C1 (MIDI 24)—fits separate bass and treble inharmonicity slopes across the bridge transition at C3 and restores full 88-key capture on small uprights and spinets [1]. Furthermore, if future iterations of the application require dense 8-to-10-note free-form polyphonic chord transcription without ghost notes, developers should avoid loading heavy ONNX Runtime or TFLite models on the audio thread [20]. Instead, keeping the C++ Gated Cascade as the ultra-fast front-end feature extractor and feeding its candidate partial power matrix candPartialPower_[128][6] into a compact, compile-time RTNeural multilayer perceptron or 16-unit LSTM (RTNeural::ModelT) provides an ideal hybrid extension [37]. Because RTNeural utilizes compile-time template unrolling and SIMD backends (XSIMD and Eigen) with zero runtime heap allocations, it executes 2–10× faster than ONNX Runtime inside hard real-time C++ audio threads, suppressing harmonic ghost notes while preserving sub-10 ms responsiveness [36].

9. Works Cited

1. README.md — Piano Engine Baseline Architecture and Benchmark Suite.

2. React Native Development Company | TantraDev, https://www.tantradev.com/technologies/react-native

3. Top React Native Interview Questions & Answers 2026, https://www.techinterview.net/questions/react-native-interview-questions

4. High Performance Game Audio with Oboe - Android Developers Blog, https://android-developers.googleblog.com/2021/03/high-performance-game-audio-with-oboe.html

5. AVAudioSession.h - GitHub, https://github.com/theos/sdks/blob/master/iPhoneOS10.3.sdk/System/Library/Frameworks/AVFoundation.framework/Frameworks/AVFAudio.framework/Headers/AVAudioSession.h

6. veetihietasalo/MiniHFT: Ultra-low latency market making engine in C++, https://github.com/veetihietasalo/MiniHFT

7. Lock-Free SPSC Queue - sinkinben, https://www.cnblogs.com/sinkinben/p/17949761/spsc-queue

8. Recently Active 'avaudiosession' Questions - Stack Overflow, https://stackoverflow.com/questions/tagged/avaudiosession?tab=Active&page=10

9. 2 Pitch analysis | Automatic Music Transcription - Rand Asswad, https://rand-asswad.xyz/muallef/2_pitch.html

10. I R I S A — Musical Acoustics and Pitch Spacing, https://perso.telecom-paristech.fr/rbadeau/assets/irisa-09.pdf

11. Optimizing a lock-free ring buffer - Hacker News, https://news.ycombinator.com/item?id=47501875

12. Low Level Fundamentals: Lock Free Programming and Atomics, https://yash30201.substack.com/p/low-level-fundamentals-lock-free

13. Lock-Free SPSC Ring Buffers in Low-Latency Trading Interviews - HackerPrep Blog, https://hackerprep.io/blog/lock-free-spsc-ring-buffers-in-low-latency-trading-interviews-c-what-interviewers-look-for-and-how-to-reason-about-memory-ordering

14. cppweeklynews/posts/202.md at dev - GitHub, https://github.com/wanghenshui/cppweeklynews/blob/dev/posts/202.md

15. Implementing a Lock-Free SPSC Queue from Scratch - CSDN, https://blog.csdn.net/cookies_s_/article/details/160229302

16. The Audio Processing Dog House - objc.io, https://www.objc.io/issue-24/audio-dog-house.html

17. Keep an eye on CycFi's 'Hz' pitch detection project; 'Q' DSP library, https://forum.pjrc.com/index.php?threads/suggestion-audio-library-keep-an-eye-on-cycfis-hz-pitch-detection-project-q-dsp-library.75957/

18. PYIN: A fundamental frequency estimator using probabilistic threshold distributions, https://www.researchgate.net/publication/269295422_PYIN_A_fundamental_frequency_estimator_using_probabilistic_threshold_distributions

19. Alexandre R.J. Francois, 'Resonate: Efficient Low Latency Spectral Analysis', https://alexandrefrancois.org/assets/publications/FrancoisARJ-ICMC2025.pdf

20. Accurate and Lightweight Instrument-Agnostic Music Transcription, https://arxiv.org/pdf/2609.04640

21. Translating Machine Learning Pipelines to Android - Arm Developer, https://developer.arm.com/community/arm-community-blogs/b/ai-blog/posts/translating-machine-learning-pipelines-to-android

22. A Lightweight Instrument-Agnostic Model for Polyphonic Note Transcription and Multipitch Estimation, https://www.researchgate.net/publication/360792948_A_Lightweight_Instrument-Agnostic_Model_for_Polyphonic_Note_Transcription_and_Multipitch_Estimation

23. CrispStrobe/CrispASR: C++ ggml runtime hub for multilingual ASR, https://github.com/CrispStrobe/CrispASR

24. Daily Papers - Hugging Face (loudness control), https://huggingface.co/papers?q=loudness%20control

25. End-to-End Deep Learning for Audio Classification - Espace ETS, https://espace.etsmtl.ca/id/eprint/2895/1/ABDOLI_Sajjad.pdf

26. Vocal Interactivity in-and-between Humans, Animals and Robots, https://vihar-2024.vihar.org/assets/VIHAR_2024_proceedings.pdf

27. Elegant Manual - TuneLab World, https://www.tunelab-world.com/TuneLabPro5.1Manual.pdf

28. Multiple Fundamental Frequency Estimation by Summing Harmonic Amplitudes - Klapuri, https://www.semanticscholar.org/paper/Multiple-Fundamental-Frequency-Estimation-by-Klapuri/a54abe400902619e851c2f86f79018f708f501ef

29. Multiple Fundamental Frequency Estimation by Summing Harmonic Amplitudes (ISMIR 2006), https://archives.ismir.net/ismir2006/paper/000125.pdf

30. Exploring Polyphonic Piano Transcription as an Inverse Problem, https://epub.jku.at/download/pdf/8025589.pdf

31. Signal to Noise Ratio Formula - GeeksforGeeks, https://www.geeksforgeeks.org/physics/signal-to-noise-ratio-formula/

32. Real-Time Pitch Estimation in Music Using Frequency Convolution, https://www.mdpi.com/2079-9292/15/17/3961

33. Processing Music Signals Using Audio Decomposition Techniques, https://audiolabs-erlangen.de/content/05_fau/professor/00_mueller/01_group/2016_Driedger_AudioDecomposition_PhD-Thesis.pdf

34. Inharmonicity - TuneLab World, https://www.tunelab-world.com/tlpihcalc.html

35. A question about Piano Meter's inharmonicity graph, https://forum.pianoworld.com/ubbthreads.php/topics/3560337/a-question-about-piano-meters-inharmonicity-graph.html

36. RTNeural real-time neural inference UGen - scsynth, https://scsynth.org/t/rtneural-real-time-neural-inference-ugen/10074

37. jatinchowdhury18/RTNeural: Real-time neural network inferencing, https://github.com/jatinchowdhury18/RTNeural

38. Add support for NAM files · Issue #143 · jatinchowdhury18/RTNeural, https://github.com/jatinchowdhury18/RTNeural/issues/143

39. RTNeural: Fast Neural Inferencing for Real-Time Systems - arXiv, https://arxiv.org/html/2106.03037v1